02/04/95

ASSEMBLY LANGUAGE/MACHINE LANGUAGE/CODING TUTORIAL  --  Part ONE

BY SCATT

INDEX

-----

PREFIX

Part ONE : The Basics - An Introduction.

First Steps

Processors

Bits and Bytes!

Number Systems

A little on Computer MEMORY

ASCII

REGISTERS

Instruction Cycle

Machine Language

ASSEMBLERS

APPENDIX A: COMMODORE 64  MEMORY MAP  ROM/RAM

APPENDIX B: Commodore 64  ROM Memory Map Routines

APPENDIX C: C64 KERNEL call addresses

APPENDIX D: OPCODES

APPENDIX E: C64 Kernal Jump Table

APPENDIX F: BASIC KEYWORDS

APPENDIX G: REU'S

APPENDIX H: ABOUT THE PROCESSOR CHIP

APPENDIX I: DIFFERENCES IN PROCESSORS

APPENDIX J: CHIP INFORMATION CHART

APPENDIX K: SPECIFICATIONS OF THE COMMODORE 64

BIBLIOGRAPHY

PREFIX

------

Hello Everyone..

This is my attempt to catalog everything I learn about Machine Language

(referred also as Machine Language and Coding) and put it into a simple

format for everyone who is interested in learning to try it out for

themselves.  Every program that I have seen has been a bit too much for

me to comprehend, and too far advanced for me. So this is my attempt to

teach Assembly Language for the Commodore 64.  Good luck, and if you want

to reach me for questions, please contact me at

ex240@cleveland.freenet.edu or as327@freenet.buffalo.edu

I AM NOT A PROGRAMMER!  ALL I KNOW AS OF THIS VERY MOMENT

IS A SMALL AMOUNT OF BASIC, so please, Don't assume I know what I am

talking about.  Let's just hope that the sources that I took all of this

data from were accurate.  If you have something to dispute about this,

please e-mail me, and I will try to make updates.  If you learn anything

new, that is not documented within the scope of this document, please,

write to me, and we'll see what we can find out TOGETHER!

Regards, SCATT

PS: If you see a number in parenthesis after a quote (i.e. "text"(4) ),

this means that the preceding text was taken from another source.  Look

at the Bibliography in the end of this text file for the source.

--------------------------------------------------------

Part ONE : The Basics - An Introduction.

THE MAIN REASON for learning Assembly or ML is this: It is FASTER, and

SMALLER (memory-wise) then BASIC programs (which stands for Beginners

All-purpose Symbolic Instruction Code), and (ML Programs) give you an

insight to how the computer operates. And best of all, It brings us

CLOSER to the computer (which is every computer geek's goal!) haha..

"THE BEST WAY TO LEARN ANY PROGRAMMING LANGUAGE IS TO PROGRAM IN

THAT LANGUAGE."(7)

"BASIC might be compared to a reliable, comfortable car.  It will take

you where you want to go.  Machine language is like a sleek racing car -

you get there with lots of time to spare.  When programming involves

large amounts of data, music, graphics, or games - speed can become the

single most important factor."(2)

"So, which language is best? (BASIC or ML) They are both best - but for

different purposes.  Many programmers, after learning ML, find that they

continue to construct programs in BASIC, and then add ML modules where

speed is important.  But perhaps the best reason of all for learning ML

is that it is fascinating and fun."(2) :)

OK let me tell you one other thing before we start.  I assume (making an

ASS out of U and ME) that you understand how your computer basically

works.  I am not going to attempt to "take a quick tour of the computers

internal parts," so please go get a book about this, ok?  :)

There are definitions all over this thing (so TAKE NOTES!!) to explain

some of the terms but that's as far as I'm gonna go with it. An example

of what you should already know is like what exactly memory is!  What is

memory? It is actually little switches and each one can have two states:

on or off!  Did you know that? IF NOT, then this is not for you!  Well,

not yet that is! Do you know what I/O, ROM, RAM, etc is?  IF NOT, again,

this is not for you YET!  You need to start out elsewhere!  I don't mean

to be rude, but we all have our starting points!  OK?  Now SMILE! And do

what must be done in order to get up to this point.  Machine Language

programming is not something to rush into...

There are A LOT of books around this wide planet, so whether you get your

information from comp.cbm or a library, or whatever, ask people! Visit

your library!  GO! GO NOW!  Don't wait another minute or else it's gonna

be too late!!!!!!!!!                :)

First Steps

-----------

I would recommend that you either get a Commodore 64 (If you don't

already have one) or a good emulator program.  One emulator I recommend

is C64S.  Ask around, especially on IRC #c-64.  They should all know

where to get it.

Once you have your C64 or emulator, I recommend you get an Assembler.

Again, ask around.  You will have one in no time.

One other thing: "Many of the first home computerists in the 1970's

learned ML before they learned BASIC.  This is because an average version

of the BASIC language used in microcomputers takes up around 12,000 bytes

of memory, and early personal computers (KIM, AIM, etc.) were severely

restricted by containing only a small amount of available memory.  These

early machines were unable to offer BASIC, so everyone programmed in

ML."(2) So hey! ML is not more difficult to understand than BASIC.  (But

sometimes more of a challenge to debug)  But it's not too far beyond

BASIC.  So DIG IN ALREADY!

Processors

----------

Another thing:  I'm not sure which processor is in the different versions

of the C=64.  I have seen 6502, and 6510.  When I figure it out, I will

update this again!  As of this point, I am not sure that all of the

commands in this book will work on the C=64.  We will learn together

though, won't we!

Well, I found some more info on the CPU.  "The heart of your machine

(C=64) is the 40-pin chip just to the left of the RF modulator can.  (He

is talking about the old-style case) This is the 6510A microprocessor."(4)

He also states that "This 40-pin custom chip operates like a 6502 MPU

(also known as CPU) except the 6510 has a built-in 6-bit peripheral I/O

port that controls memory management and cassette I/O."

Bits and Bytes!

--------------

"It's interesting that the word "bit" is frequently explained as a

shortening of the phrase BInary digiT.  In fact, the word bit goes back

several centuries.  There was a coin which was soft enough to be cut

with a knife into eight pieces.  Hence, pieces of eight.  A single piece

of this coin was called a bit and, as with computer memories, it meant

that you couldn't slice it any further.  We still use the word bit today

as in the phrase "two bits" meaning 25 cents."(2)

A byte is 8 bits of data that may be loaded together into a register.  A

register holds 1 byte.  The 6502 can only affect 1 byte in one

operation.  Because the 6502 can perform hundreds of thousands of

operations a second, it can affect 100's of 1000's of bytes per second.

In fact, "the Commodore 64 can handle about 500,000 of these steps each

second." This is from the C-64 Troubleshooting & Repair Guide by Robert

C. Brenner.

Number Systems

----------------------

DECimal Numbers:  We all know what these are, like 0,1,2,3 etc.  These

are base 10 numbers.  ML can be accomplished in Decimal, but very rarely

seen.

*BINary Numbers: Binary numbers are base 2 numbers.  All we have to

remember in Binary numbers is 0's and 1's.  It's supposedly how the

computer "thinks".  What I take this as is that it's the way the

processor sends and receives data internally (through it's 8-bit

channel.) with 1's (or positive voltage) and 0's or a lack of voltage.

All digits and numbers are converted to BIN.  The easiest way to convert

DECimal numbers to Binary is this:

Place      0 0 0 0    Here we have 1's place, 2's place,

Holder->  8 4 2 1    4's place and 8's place and so on..

-------

Bin Num->  0 0 0 0    Here's the binary number..

So, if we have a binary number of let's say, 0101, then we  just add up

the place's numbers and see what decimal number  we get..  So we have a 1

in the 4's place, so that's decimal  #4.  We have no 8's or 2's and we

have 1 in the 1's place.  So if we add the 4 to the 1, we get a decimal

of 5.  So, if  we had let's say a decimal number of like 12, we would

know  that there is at least one 8, and a 4, and we come up with

1100(bin)=12(dec)!  Try some on your own and get familiar  converting

these back and fourth.....

BINARY         DECIMAL         BINARY          DECIMAL

------         -------         ------          -------

0000           0               0110            6

0001           1               0111            7

0010           2               1000            8

0011           3               1001            9

0100           4               1010            10

0101           5               1011            11

The Bit significance and the byte..

Bit Number:        b7  b6  b5  b4  b3  b2  b1  b0

Bit Significance:  128 64  32  16  8   4   2   1

Binary Number:     0   0   0   0   0   0   0   0

This would be an 8-BIT Binary number.  Often written as 0000 0000.

Understood? Kool. So the Decimal number "25" would convert to what? Yup,

you got it, 0001 1001 !!!

The rightmost Bit=Bit 0 (Tells us whether we have a 1 in our byte) The

next to the left (Bit 1) tells us whether we have a two, etc..

And we go ON!

*HEX Numbers: Hexadecimal Numbers are Base 16. "HEX" for 6, and DECI for

10, so when you add them, 6+10=16!!!  :) Kool. That is, multiples of 16.

0,1,2,3,4,5,6,7,8,9,a,b,c,d,e,f.  When we program (or the new word seems

to be "code" or shall I say the "in" word haha..) So when we CoDe, we use

a "$" to represent HEX numbers. Remember this.  Put it into your ROM and

KEEP IT THERE!  It is important!

"See how hex $10 (see the dollar-sign?) looks like binary?  If you split

a hex number into two parts, 1 and 0, and the binary (it's an eight-bit

group, a byte) into two parts, 0001 and 0000 - you can see the

relationship."(2)

Remember when I did this: 0000 0000? Well, some people consider one of

those sets of 4 bits to be a "nybble".  To represent a byte (8-bits) in

HEX notation, divide the 8-bit byte into two 4-bit units (yup, that's a

nybble).  Each of the 4-bit units (or nybbles) has a value of from 0 to

15 (decimal) which we express with a single hexadecimal digit!  So you

can use just ONE hexadecimal digit to represent 1 nybble (4-bits)! Isn't

that kool! Now you remembered that the "$" represents the HEX notation,

right? Well, check out this chart:

HEX                     DECIMAL

---                     ----

$0      =               0

$01     =               1

$02     =               2

$03     =               3

$04     =               4

$05     =               5

$06     =               6

$07     =               7

$08     =               8 (gee this gets boring..)

$09     =               9

$0A     =               10 (what's this?   WO! an "A"!!!)

$0B     =               11

$0C     =               12

$0D     =               13

$0E     =               14

$0F     =               15

$10     =               16

$11     =               17

$12     =               18

$13     =               19

etc

etc

So there we have it..

Here's another way to put it:

"	DECIMAL	0 1 2 3 4 5 6 7 8 9   then you start over with 10

HEX	00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F then you

start over with 10"(2)

Let me go and see if I can find some text on how to mathematically

convert decimal to hex.. I'll be right back..

Well, I didn't find what I was looking for, but I found this little charm..

"Microsoft Hex-Decimal Converter"(2)

1	HE$="0123456789ABCDEF"

2	?"{CLEAR}{03 DOWN}PLEASE CHOOSE:

4	?"{03 DOWN}{03 RIGHT}1-INPUT HEX & GET DECIMAL BACK.

5 	REM NEW LINE HERE

6	?"{02 DOWN}{03 RIGHT}2-INPUT DECIMAL TO GET HEX BACK.

7	GET K:IF K=0 THEN GOTO 7

9	?"{CLEAR}":ON K GOTO 200,400

100	H$="":FOR M=3 TO 0 STEP -1:N%=DE/(16^M):

DE=DE-N%*16^M:H$=H$+MID$(HE$,N%+1,1):NEXT

101 	RETURN

102	D=0:Q=3:FOR M=1 TO 4:FOR W=0 TO 15:

IF MID$(H$,M,1)=MID$(HE$,W+1,1) THEN GOTO 104

103	NEXT W

104	D1=W*(16^(Q)):D=D+D1:Q=Q-1:NEXT M

105	DE=INT(D):RETURN

200	INPUT"{02 DOWN}HEX";H$:GOSUB 102:

PRINT SPC(11)"{UP}= {REV}"DE"{LEFT} "

210	GOTO 200

400	INPUT"{02 DOWN}DECIMAL";DE:GOSUB 100:

PRINT SPC(14)"{UP}= {REV} "H$" "

410	GOTO 400

Something useful: "To figure out a HEX number, multiply the second column

by 16 and add the other number to it.  So, $1A would be one times 16 plus

10 (Recall that A stands for ten)."(2)

Well, since I sent in my $$ to register "The PC Assembler Tutor" and never got

anything back from the guy, I will ASSUME (ASS-U-ME) that Mr. Nelson

won't mind me reproducing this next goody without his consent. (Although

I did mention his name to keep him happy! :)

HEX             CONVERT       BINARY

---             -------       -------

"3         ->   2 + 1     ->   0011

9         ->   8 + 1     ->   1001

F = 15    ->   8+4+2+1   ->   1111

All computers operate on binary data, so why do we use hex

numbers? Take a test. Copy these two binary numbers:

1011 1000 0110 1010 1001 0101 0111 1010

0111 1100 0100 1100 0101 0110 1111 0011

Now copy these two hex numbers:

B86A957A

7C4C56F3

As you can see, you recognize hex numbers faster and you make

fewer mistakes in transcription with hex numbers.

ADDITION AND SUBTRACTION

The rules for binary addition are easy:

0 + 0 = 0

0 + 1 = 1

1 + 0 = 1

1 + 1 = 0  (carry 1 to the next digit left)

similarly for binary subtraction:

0 - 0 = 0

0 - 1 = 1  (borrow 1 from the next digit left)

1 - 0 = 1

1 - 1 = 0" (8)

OK.. I hope that clears some stuff up.. Well, for now, I can't find much

on converting Decimal numbers to Hex, so as the book states "Even the

sketchiest understanding of hexadecimal math, however, should be

sufficient for you to follow and use (assembly)"(1)

and...

"You need not memorize (HEX NUMBERS) beyond learning to count from 1 to

16 - learning the symbols.  Be able to count from 00 up to 0F. (By

convention, even the smallest hex number is listed as two digits as in 03

or 0B."(2)

So, what I would recommend you do (and what I will be doing before not

too long) is copying a DEC to HEX table from somewhere (or just make your

own) and tape it in front of you, avoiding the monitor you are using for

a billboard, and you will then know how to convert DEC to HEX or visa

versa.

As I've heard somewhere before, and also very useful, "Most ML programming

involves working with hex numbers only between 0 and 255.  This is

because a single byte (8-bits) can hold no number larger than 255.

Manipulating numbers larger than 255 is no real importance in ML

programming until you are ready to work with more advanced ML programs.

For example, all 6502 ML instructions are coded into one byte, all the

"flags" are held in one byte, and many "addressing modes" use one byte to

hold their argument."(2)

A little on Computer MEMORY

---------------------------

I'm sorry to use so many quotes, but everything I've found seems so

useful, and I am learning so much from all of this info, I just can't

stop!  And all the typing is very good for my fingers..

"THE CITY OF BYTES

Imagine a city with a single long row of houses.  It's night.  Each house

has a peculiar Christmas display: on the roof is a line of eight lights.

The houses represent bytes; each light is a single bit.  If we fly over

the city of bytes, at first we see only darkness.  Each byte contains

nothing (zero), so all eight of its bulbs are off. (On the horizon we can

see a glow, however, because the computer has memory up there, called ROM

memory, which is very active and contains built-in programs.) But we are

down in RAM, our free user-memory, and there are no programs now in RAM,

so every house is dark.  Let's observe what happens to an individual byte

when different numbers are stored there; we can randomly choose byte

1504.  We hover over that house to see what information is "contained" in

the light display.

____.____.____.____.____.____.____.____.____("."=off, "o"=on)

Like all the rest, this byte is dark.  Each bulb is off.  Observing this,

we know that the byte here is "holding" or representing a zero.  If

someone at the computer types in POKE 1504,1 - suddenly the rightmost

light bulb goes on and the byte holds a one instead of a zero:

____.____.____.____.____.____.____.____o____

This rightmost bulb is in the 1's column (just as it would be in our

usual way of counting by ten's, our familiar decimal system). But the

next bulb is in a 2's column, so POKE 1504,2 would be:

____.____.____.____.____.____.____o____.____

And three would be one and two:

____.____.____.____.____.____.____o____o____

In this way - by checking which bits are turned on and then adding them

together - the computer can look at a byte and know what number is

there.  Each light bulb, each BIT, is in its own special position in the

row of eight and has a value twice the value of the one just before it:

____o____o____o____o____o____o____o____o____ = 255!

128's 64's 32's 16's 8's  4's  2's  1's

65535 is an interesting number because it represents the limit of our

computer's memories.  In special cases, with additional hardware, memory

can be expanded beyond this.  But this is the normal upper limit because

the 6502 chip is designed to be able to address (put bytes in or take

them out of memory cells) up to $FFFF."(2)

ASCII

-------

"Instead of a number from 0 to 255, an 8-bit byte can be used to

represent an upper or lower case letter of the alphabet, a punctuation

mark, or a printer-control character such as a carriage return."(1)

ASCII-American Standard Code for Information Interchange. You've heard it

a million times, and will hear it a million more. It is the "closest

thing the industry has to a standard set of character codes."(1)  So,

"Whether a given byte is interpreted as a number, an ASCII character, or

something else depends entirely on the program using that byte."(1)

REGISTERS

---------------

A register is a special area in memory for storing the data upon which

the program is operating.

Three Registers in the 6502 Processor:

A- Accumulator - Can add or subtract any number up to 255

X, and Y - These can either be used to add one or subtract

one digit.

"  The "A" register is often called the accumulator which indicates its

function: all math and logical manipulations are done to the "A" register (from

here on out it will be referred to as .A).

There are two other registers inside the 6502 processor, specifically .X and

.Y.  These registers help act as counters and indexes into memory (sort of like

mem[x] in pascal but not quite...)."(7)

The 6502 can set one register equal to any other register.

Instruction Cycle

---------------------

*The 6502 only knows 151 instructions called opcodes. (I'm not sure if

this has changed in the C=64, but I will find out. and update this) Each

opcode is 1-byte (8-bits) long. Opcodes tell the processor what to do.

The processor gets the first opcode, preforms the specified operation,

gets the next opcode, preforms the operation, etc.

So where does the processor get the list of opcodes? You got it, from the

program.  The 6502 has a PC (Program Counter) that tells it where to get

the next opcode from in memory.  The PC stores the address of some

location in memory.  When the processor starts it's instruction cycle, it

looks at the PC, gets the memory location for the first op- code, goes

there, and preforms the operation specified by that opcode. When it's

done with the first one, it MAKES the PC point to the next opcode.  So

the processor uses the PC as sort of a MAP.  Then, it again looks at the

PC and gets the memory location back and goes there and starts over again.

Here's a cool flowchart:

[---------------------------]

[  Fetch opcode pointed to  ]

[  by the PC (Program       ]<-----\

[  counter.                 ]      |

[---------------------------]      |

|                    |

\|/                   |

[---------------------------]      |

[  Perform operation        ]      |

[  specified by opcode      ]      |

[---------------------------]      |

|                    |

\|/                   |

[---------------------------]      |

[  Make PC (Program Counter)]      |

[  point to next opcode in  ]      |

[        memory             ]      |

[---------------------------]      |

|                    |

|____________________|

Cool, eh?

This is the 6502 Instruction Cycle.

MACHINE LANGUAGE

----------------------

Machine Language program is nothing more then a series of ML instructions

stored in memory.  Each ML instruction is stored in memory as a 1-byte

(8-bit) long opcode which may be followed by 1 or 2 bytes of operand.  ML

is usually in hexadecimal format. So, here is a short ML program: A9 05

20 02 04 A2 F5 60 Yup. Just a bunch of numbers! cool.

ASSEMBLERS

------------

"To make it easier to write programs in machine language (called "ML"

from here on), most programmers use a special program called an

assembler.  This is where the term "assembly language" comes from.  ML

and assembly language programs are both essentially the same thing.

Using an assembler to create ML programs is far easier than being forced

to look up and then POKE each byte into RAM memory.  That's the way it

used to be done, when there was too little memory in computers to hold

languages (like BASIC or Assemblers) at the same time as programs created

by those languages.  That old style hand-programming was very

laborious."(2)

"Program (which) takes source code in basic form or from a file and

writes to memory or a file the resulting executable. Allows higher

flexibility than a monitor (see below) due to use of labels etc and not

having to keep track of each address within the program.

Monitor - A program, resident in memory, invoked by a SYS call from basic

or by hitting the restore key that will let you disassemble, assemble and

examine areas of memory and execute programs directly from the monitor.

Useful for debugging programs and for writing short programs."(7)

One monitor that I've seen is the MLX monitor.

Object Code: is a series of 6502 machine language instructions to be

stored in memory and executed.

Source Code: An assembly language source program consists of one or more

lines of assembly language source code.  These consist of 4 fields:

LABEL ---- MNEMONIC ---- OPERAND ---- COMMENT

Label is a name given to the instruction.  Similar to BASIC line numbers.

Mnemonic is a cool word!  It is the 3-letter name that suggests a

function of a given ML instruction. (Easy! -- like LDA, LDX, or LDY...

we'll get into these later.)

Operand would be the action of the Mnemonic.  It's like this:

LDA	$0300 <---operand... in this case we're loading

the accumulator with $0300..

LABEL- This is an optional field.  This is where you put your comments.

You separate the Label from the rest of the instruction with a ";"

(semicolon)..  This makes the source code more understandable.

Here's another cool flowchart:

Source of input-->    	PROGRAMMER

|

\|/

What he/she inputs--> 	SOURCE CODE

|

\|/

Program that converts 	  --->	ASSEMBLER

Source code to ML	     |

/ \

/   \

/     \

/       \

Output:	   Assembler    Object

listing	 code

|	   |

Intended for the:    Programmer   Processor

OK!  Now if any of this is a bit confusing, look it over, and get used to

it!  You will be responsible for having this stuff in the back of your

head at ALL TIMES!!! Good luck.. Next up is some Mnemonics!

See you all then!

APPENDIX A

----------

COMMODORE 64  MEMORY MAP  ROM/RAM

; Data types in headers (for reassembler):

;

;	DATA		Misc data

;	TEXT		String terminated with 00

;	WORD		Vectors in LO/HI byte pairs

;	CHIP		I/O Area

;	EMPTY		ROM containing FF's or AA's

;

HEX	DECIMAL		BITS	DESCRIPTION

0000	0		7-0	MOS 6510 Data Direction

Register (xx101111)

Bit= 1: Output, Bit=0:

Input, x=Don't Care

0001	1			MOS 6510 Micro-Processor

On-Chip I/O Port

0	/LORAM Signal (0=Switch	BASIC ROM Out)

1	/HIRAM Signal (0=Switch Kernal ROM Out)

2	/CHAREN Signal (O=Swith Char. ROM In)

3	Cassette Data Output Line

4	Cassette Switch Sense: 1 = Switch Closed

5	Cassette Motor Control

O = ON, 1 = OFF

6-7	Undefined

D6510     0000        0       6510 On-chip Data Direction Register.

R6510     0001        1       6510 On-chip 8-bit Input/Output Register.

TEMP      0002        2       Unused. Free for user programs.

ADRAY1    0003-0004   3       Jump Vector: Convert FAC to Integer in (A/Y)

($B1AA).

ADRAY2    0005-0006   5       Jump Vector: Convert Integer in (A/Y) to

Floating point in (FAC); ($B391).

CHARAC    0007        7       Search Character/Temporary Integer during INT.

ENDCHR    0008        8       Flag: Scan for Quote at end of String.

INTEGR    0007-0008   7       Temporary Integer during OR/AND.

TRMPOS    0009        9       Screen Column for last TAB.

VERCK     000A        10      Flag: 0 = Load, 1 = Verify.

COUNT     000B        11      Input Buffer Pointer/Number of Subscripts.

DIMFLG    000C        12      Flag: Default Array dimension.

VALTYP    000D        13      Data type Flag: $00 = Numeric, $FF = String.

INTFLG    000E        14      Data type Flag: $00 = Floating point, $80 =

Integer.

GARBFL    000F        15      Flag: DATA scan/List Quote/Garbage collection.

SUBFLG    0010        16      Flag: Subscript reference/User Function call.

INPFLG    0011        17      Input Flag: $00 = INPUT, $40 = GET, $98 =

READ.

TANSGN    0012        18      Flag: TAN sign/Comparative result.

CHANNL    0013        19      File number of current Input Device.

LINNUM    0014-0015   20      Temporary: Integer value.

TEMPPT    0016        22      Pointer: Temporary String Stack.

LASTPT    0017-0018   23      Last temporary String Address.

TEMPST    0019-0021   25      Stack for temporary Strings.

INDEX     0022-0025   34      Utility Pointer Area.

INDEX1    0022-0023   34      First Utility Pointer.

INDEX2    0024-0025   36      Secong Utility Pointer.

RESHO     0026-002A   38      Floating point product of Multiply and

Divide.

TXTTAB    002B-002C   43      Pointer: Start of BASIC Text Area ($0801).

VARTAB    002D-002E   45      Pointer: Start of BASIC Variables.

ARYTAB    002F-0030   47      Pointer: Start of BASIC Arrays.

STREND    0031-0032   49      Pointer: End of BASIC Arrays + 1.

FRETOP    0033-0034   51      Pointer: Bottom of String space.

FRESPC    0035-0036   53      Utility String Pointer.

MEMSIZ    0037-0038   55      Pointer: Highest Address available to BASIC

($A000).

CURLIN    0039-003A   57      Current BASIC Line number.

OLDLIN    003B-003C   59      Previous BASIC Line number.

OLDTXT    003D-003E   61      Pointer: BASIC Statement for CONT.

DATLIN    003F-0040   63      Current DATA Line number.

DATPTR    0041-0042   65      Pointer: Used by READ - current DATA Item

Address.

INPPTR    0043-0044   67      Pointer: Temporary storage of Pointer during

INPUT Routine.

VARNAM    0045-0046   69      Name of Variable being sought in Variable

Table.

VARPNT    0047-0048   71      Pointer: to value of (VARNAM) if Integer, to

descriptor if String.

FORPNT    0049-004A   73      Pointer: Index Variable for FOR/NEXT loop.

VARTXT    004B-004C   75      Temporary storage for TXTPTR during READ,

INPUT and GET.

OPMASK    004D        77      Mask used during FRMEVL.

TEMPF3    004E-0052   78      Temporary storage for FLPT value.

FOUR6     0053        83      Length of String Variable during Garbege

collection.

JMPER     0054-0056   84      Jump Vector used in Function Evaluation -

JMP followed by Address ($4C,$LB,$MB).

TEMPF1    0057-005B   87      Temporary storage for FLPT value.

TEMPF2    005C-0060   92      Temporary storage for FLPT value.

FAC       0061-0066   97      Main Floating point Accumulator.

FACEXP    0061        97      FAC Exponent.

FACHO     0062-0065   98      FAC Mantissa.

FACSGN    0066        102     FAC Sign.

SGNFLG    0067        103     Pointer: Series Evaluation Constant.

BITS      0068        104     Bit Overflow Area during normalisation

Routine.

AFAC      0069-006E   105     Auxiliary Floating point Accumulator.

ARGEXP    0069        105     AFAC Exponent.

ARGHO     006A-006D   106     AFAC Mantissa.

ARGSGN    006E        110     AFAC Sign.

ARISGN    006F        111     Sign of result of Arithmetic Evaluation.

FACOV     0070        112     FAC low-order rounding.

FBUFPT    0071-0072   113     Pointer: Used during CRUNCH/ASCII conversion.

CHRGET    0073-008A   115     Subroutine: Get next Byte of BASIC Text.

,0073  INC $7A       ,0082  BEQ $0073

,0075  BNE $0079     ,0084  SEC

,0077  INC $7B       ,0085  SBC #$30

! ,0079  LDA $0801     ,0087  SEC

,007C  CMP #$3A      ,0088  SBC #$D0

,007E  BCS $008A     ,008A  RTS

,0080  CMP #$20

CHRGOT    0079        121     Entry to Get same Byte again.

TXTPTR    007A-007B   122     Pointer: Current Byte of BASIC Text.

RNDX      008B-008F   139     Floating RND Function Seed Value.

STATUS    0090        144     Kernal I/O Status Word  ST.

STKEY     0091        145     Flag: $7F = STOP key.

SVXT      0092        146     Timing Constant for Tape.

VERCKK    0093        147     Flag: 0 = Load, 1 = Verify.

C3PO      0094        148     Flag: Serial Bus - Output Character buffered.

BSOUR     0095        149     Buffered Character for Serial Bus.

SYNO      0096        150     Cassette Sync. number.

TEMPX     0097        151     Temporary storage of X Register during CHRIN.

TEMPY     0097        151     Temporary storage of Y Register during RS232

fetch.

LDTND     0098        152     Number of Open Files/Index to File Table.

DFLTN     0099        153     Default Input Device (0).

DFLTO     009A        154     Default Output Device (3).

PRTY      009B        155     Parity of Byte Output to Tape.

DPSW      009C        156     Flag: Byte received from Tape.

MSGFLG    009D        157     Flag: $00 = Program mode: Suppress Error

Messages, $40 = Kernal Error Messages only,

$80 = Direct mode: Full Error Messages.

FNMIDX    009E        158     Index to Cassette File name/Header ID for

Tape write.

PTR1      009E        158     Tape Error log pass 1.

PTR2      009F        159     Tape Error log pass 2.

TIME      00A0-00A2   160     Real-time jiffy Clock (Updated by IRQ

Interrupt approx. every 1/60 of Second);

Update Routine: UDTIMK ($F69B).

TSFCNT    00A3        163     Bit Counter Tape Read or Write/Serial Bus

EOI (End Of Input) Flag.

TBTCNT    00A4        164     Pulse Counter Tape Read or Write/Serial Bus

shift Counter.

CNTDN     00A5        165     Tape Synchronising count down.

BUFPNT    00A6        166     Pointer: Tape I/O buffer.

INBIT     00A7        167     RS232 temporary for received Bit/Tape

temporary.

BITC1     00A8        168     RS232 Input Bit count/Tape temporary.

RINONE    00A9        169     RS232 Flag: Start Bit check/Tape temporary.

RIDATA    00AA        170     RS232 Input Byte Buffer/Tape temporary.

RIPRTY    00AB        171     RS232 Input parity/Tape temporary.

SAL       00AC-00AD   172     Pointer: Tape Buffer/Screen scrolling.

EAL       00AE-00AF   174     Tape End Address/End of Program.

CMPO      00B0-00B1   176     Tape timing Constants.

TAPE1     00B2-00B3   178     Pointer: Start Address of Tape Buffer ($033C).

BITTS     00B4        180     RS232 Write bit count/Tape Read timing Flag.

NXTBIT    00B5        181     RS232 Next Bit to send/Tape Read - End of

Tape.

RODATA    00B6        182     RS232 Output Byte Buffer/Tape Read Error Flag.

FNLEN     00B7        183     Number of Characters in Filename.

LA        00B8        184     Current File - Logical File number.

SA        00B9        185     Current File - Secondary Address.

FA        00BA        186     Current File - First Address (Device number).

OPEN LA,FA,SA;  OPEN 1,8,15,"I0":CLOSE 1

FNADR     00BB-00BC   187     Pointer: Current File name Address.

ROPRTY    00BD        189     RS232 Output Parity/Tape Byte to be Input or

Output.

FSBLK     00BE        190     Tape Input/Output Block count.

MYCH      00BF        191     Serial Word Buffer.

CAS1      00C0        192     Tape Motor Switch.

STAL      00C1-00C2   193     Start Address for LOAD and Cassette Write.

MEMUSS    00C3-00C4   195     Pointer: Type 3 Tape LOAD and general use.

LSTX      00C5        197     Matrix value of last Key pressed; No Key = $40.

NDX       00C6        198     Number of Characters in Keyboard Buffer

queue.

RVS       00C7        199     Flag: Reverse On/Off; On = $01, Off = $00.

INDX      00C8        200     Pointer: End of Line for Input (Used to

suppress trailing spaces).

LXSP      00C9-00CA   201     Cursor X/Y (Line/Column) position at start of

Input.

SFDX      00CB        203     Flag: Print shifted Characters.

BLNSW     00CC        204     Flag: Cursor blink; $00 = Enabled, $01 =

Disabled.

BLNCT     00CD        205     Timer: Count down for Cursor blink toggle.

GDBLN     00CE        206     Character under Cursor while Cursor Inverted.

BLNON     00CF        207     Flag: Cursor Status; $00 = Off, $01 = On.

CRSW      00D0        208     Flag: Input from Screen = $03, or Keyboard =

$00.

PNT       00D1-00D2   209     Pointer: Current Screen Line Address.

PNTR      00D3        211     Cursor Column on current Line, including

Wrap-round Line, if any.

QTSW      00D4        212     Flag: Editor in Quote Mode; $00 = Not.

LNMX      00D5        213     Current logical Line length: 39 or 79.

TBLX      00D6        214     Current Screen Line number of Cursor.

SCHAR     00D7        215     Screen value of current Input Character/Last

Character Output.

INSRT     00D8        216     Count of number of inserts outstanding.

LDTB1     00D9-00F2   217     Screen Line link Table/Editor temporaries.

High Byte of Line Screen Memory Location.

USER      00F3-00F4   243     Pointer: Current Colour RAM Location.

KEYTAB    00F5-00F6   245     Vector: Current Keyboard decoding Table.

($EB81)

RIBUF     00F7-00F8   247     RS232 Input Buffer Pointer.

ROBUF     00F9-00FA   249     RS232 Output Buffer Pointer.

FREKZP    00FB-00FE   251     Free Zero Page space for User Programs.

BASZPT    00FF        255     BASIC temporary Data Area.

ASCWRK    00FF-010A   255     Assembly Area for Floating point to ASCII

conversion.

BAD       0100-013E   256     Tape Input Error log.

STACK     0100-01FF   256     6510 Hardware Stack Area.

BSTACK    013F-01FF   319     BASIC Stack Area.

BUF       0200-0258   512     BASIC Input Buffer (Input Line from Screen).

LAT       0259-0262   601     Kernal Table: Active logical File numbers.

FAT       0263-026C   611     Kernal Table: Active File First Addresses

(Device numbers).

SAT       026D-0276   621     Kernal Table: Active File Secondary

Addresses.

KEYD      0277-0280   631     Keyboard Buffer Queue (FIFO).

MEMSTR    0281-0282   641     Pointer: Bottom of Memory for Operating

System ($0800).

MEMSIZ    0283-0284   643     Pointer: Top of Memory for Operating

System ($A000).

TIMOUT    0285        645     Serial IEEE Bus timeout defeat Flag.

COLOR     0286        646     Current Character Colour code.

GDCOL     0287        647     Background Colour under Cursor.

HIBASE    0288        648     High Byte of Screen Memory Address ($04).

XMAX      0289        649     Maximum number of Bytes in Keyboard

Buffer ($0A).

RPTFLG    028A        650     Flag: Repeat keys; $00 = Cursors, INST/DEL &

Space repeat, $40 no Keys repeat, $80 all

Keys repeat ($00).

KOUNT     028B        651     Repeat Key: Speed Counter ($04).

DELAY     028C        652     Repeat Key: First repeat delay Counter ($10).

SHFLAG    028D        653     Flag: Shift Keys: Bit 1 = Shift, Bit 2 = CBM,

Bit 3 = CTRL; ($00 = None, $01 = Shift, etc.).

LSTSHF    028E        654     Last Shift Key used for debouncing.

KEYLOG    028F-0290   655     Vector: Routine to determine Keyboard table

to use based on Shift Key Pattern ($EB48).

MODE      0291        657     Flag: Upper/Lower Case change: $00 = Disabled,

$80 = Enabled ($00).

AUTODN    0292        658     Flag: Auto scroll down: $00 = Disabled ($00).

M51CTR    0293        659     RS232 Pseudo 6551 control Register Image.

M51CDR    0294        660     RS232 Pseudo 6551 command Register Image.

M51AJB    0295-0296   661     RS232 Non-standard Bits/Second.

RSSTAT    0297        663     RS232 Pseudo 6551 Status Register Image.

BITNUM    0298        664     RS232 Number of Bits left to send.

BAUDOF    0299-029A   665     RS232 Baud Rate; Full Bit time microseconds.

RIDBE     029B        667     RS232 Index to End of Input Buffer.

RIDBS     029C        668     RS232 Pointer: High Byte of Address of Input

Buffer.

RODBS     029D        669     RS232 Pointer: High Byte of Address of Output

Buffer.

RODBE     029E        670     RS232 Index to End of Output Buffer.

IRQTMP    029F-02A0   671     Temporary store for IRQ Vector during Tape

operations.

ENABL     02A1        673     RS232 Enables.

TODSNS    02A2        674     TOD sense during Tape I/O.

TRDTMP    02A3        675     Temporary storage during Tape READ.

TD1IRQ    02A4        676     Temporary D1IRQ Indicator during Tape READ.

TLNIDX    02A5        677     Temporary for Line Index.

TVSFLG    02A6        678     Flag: TV Standard: $00 = NTSC, $01 = PAL.

TEMP      02A7-02FF   679     Unused.

SPR11     02C0-02FE   704     Sprite #11 Data Area.

(SCREEN + $03F8 + SPR number)

POKE 1024+1016+0,11  to use Sprite#0 DATA

from ($02C0-$02FE).

IERROR    0300-0301   768     Vector: Indirect entry to BASIC Error

Message, (X) points to Message ($E38B).

IMAIN     0302-0303   770     Vector: Indirect entry to BASIC Input Line

and Decode ($A483).

ICRNCH    0304-0305   772     Vector: Indirect entry to BASIC Tokenise

Routine ($A57C).

IQPLOP    0306-0307   774     Vector: Indirect entry to BASIC LIST

Routine ($A71A).

IGONE     0308-0309   776     Vector: Indirect entry to BASIC Character

dispatch Routine ($A7E4).

IEVAL     030A-030B   778     Vector: Indirect entry to BASIC Token

evaluation ($AE86).

SAREG     030C        780     Storage for 6510 Accumulator during SYS.

SXREG     030D        781     Storage for 6510 X-Register during SYS.

SYREG     030E        782     Storage for 6510 Y-Register during SYS.

SPREG     030F        783     Storage for 6510 Status Register during SYS.

USRPOK    0310        784     USR Function JMP Instruction ($4C).

USRADD    0311-0312   785     USR Address ($LB,$MB).

TEMP      0313        787     Unused.

CINV      0314-0315   788     Vector: Hardware IRQ Interrupt Address ($EA31).

CNBINV    0316-0317   790     Vector: BRK Instruction Interrupt Address

($FE66).

NMINV     0318-0319   792     Vector: Hardware NMI Interrupt Address ($FE47).

IOPEN     031A-031B   794     Vector: Indirect entry to Kernal OPEN

Routine ($F34A).

ICLOSE    031C-031D   796     Vector: Indirect entry to Kernal CLOSE

Routine ($F291).

ICHKIN    031E-031F   798     Vector: Indirect entry to Kernal CHKIN

Routine ($F20E).

ICKOUT    0320-0321   800     Vector: Indirect entry to Kernal CHKOUT

Routine ($F250).

ICLRCH    0322-0323   802     Vector: Indirect entry to Kernal CLRCHN

Routine ($F333).

IBASIN    0324-0325   804     Vector: Indirect entry to Kernal CHRIN

Routine ($F157).

IBSOUT    0326-0327   806     Vector: Indirect entry to Kernal CHROUT

Routine ($F1CA).

ISTOP     0328-0329   808     Vector: Indirect entry to Kernal STOP

Routine ($F6ED).

IGETIN    032A-032B   810     Vector: Indirect entry to Kernal GETIN

Routine ($F13E).

ICLALL    032C-032D   812     Vector: Indirect entry to Kernal CLALL

Routine ($F32F).

USRCMD    032E-032F   814     User Defined Vector ($FE66).

ILOAD     0330-0331   816     Vector: Indirect entry to Kernal LOAD

Routine ($F4A5).

ISAVE     0332-0333   818     Vector: Indirect entry to Kernal SAVE

Routine ($F5ED).

TEMP      0334-033B   820     Unused.

TBUFFR    033C-03FB   828     Tape I/O Buffer.

SPR13     0340-037E   832     Sprite #13.

SPR14     0380-03BE   896     Sprite #14.

SPR15     03C0-03FE   960     Sprite #15.

TEMP      03FC-03FF   1020    Unused.

VICSCN    0400-07E7   1024    Default Screen Video Matrix.

TEMP      07E8-07F7   2024    Unused.

SPNTRS    07F8-07FF   2040    Default Sprite Data Pointers.

0800-9FFF   2048    Normal BASIC Program space.

8000-9FFF   32768   Optional Cartridge ROM space.

A000-BFFF   40960   BASIC ROM (Part) or 8 KB RAM.

a000	40960	-	Restart Vectors				WORD

a00c	40972	stmdsp	BASIC Command Vectors			WORD

a052	41042	fundsp	BASIC Function Vectors			WORD

a080	41088	optab	BASIC Operator Vectors			WORD

a09e	41118	reslst	BASIC Command Keyword Table		DATA

a129	41257	msclst	BASIC Misc. Keyword Table		DATA

a140	41280	oplist	BASIC Operator Keyword Table		DATA

a14d	41293	funlst	BASIC Function Keyword Table		DATA

a19e	41374	errtab	Error Message Table			DATA

a328	41768	errptr	Error Message Pointers			WORD

a364	41828	okk	Misc. Messages				TEXT

a38a	41866	fndfor	Find FOR/GOSUB Entry on Stack

a3b8	41912	bltu	Open Space in Memory

a3fb	41979	getstk	Check Stack Depth

a408	41992	reason	Check Memory Overlap

a435	42037	omerr	Output ?OUT OF MEMORY Error

a437	42039	error	Error Routine

a469	42089	errfin	Break Entry

a474	42100	ready	Restart BASIC

a480	42112	main	Input & Identify BASIC Line

a49c	42140	main1	Get Line Number & Tokenise Text

a4a2	42146	inslin	Insert BASIC Text

a533	42291	linkprg	Rechain Lines

a560	42336	inlin	Input Line Into Buffer

a579	42361	crunch	Tokenise Input Buffer

a613	42515	fndlin	Search for Line Number

a642	42562	scrtch	Perform [new]

a65e	42590	clear	Perform [clr]

a68e	42638	stxpt	Reset TXTPTR

a69c	42652	list	Perform [list]

a717	42775	qplop	Handle LIST Character

a742	42818	for	Perform [for]

a7ae	42926	newstt	BASIC Warm Start

a7c4	42948	ckeol	Check End of Program

a7e1	42977	gone	Prepare to execute statement

a7ed	42989	gone3	Perform BASIC Keyword

a81d	43037	restor	Perform [restore]

a82c	43052	stop	Perform [stop], [end], break

a857	43095	cont	Perform [cont]

a871	43121	run	Perform [run]

a883	43139	gosub	Perform [gosub]

a8a0	43168	goto	Perform [goto]

a8d2	43218	return	Perform [return]

a8f8	43256	data	Perform [data]

a906	43270	datan	Search for Next Statement / Line

a928	43304	if	Perform [if]

a93b	43323	rem	Perform [rem]

a94b	43339	ongoto	Perform [on]

a96b	43371	linget	Fetch linnum From BASIC

a9a5	43429	let	Perform [let]

a9c4	43460	putint	Assign Integer

a9d6	43478	ptflpt	Assign Floating Point

a9d9	43481	putstr	Assign String

a9e3	43491	puttim	Assign TI$

aa2c	43564	getspt	Add Digit to FAC#1

aa80	43648	printn	Perform [print]#

aa86	43654	cmd	Perform [cmd]

aa9a	43674	strdon	Print String From Memory

aaa0	43680	print	Perform [print]

aab8	43704	varop	Output Variable

aad7	43735	crdo	Output CR/LF

aae8	43752	comprt	Handle comma, TAB(, SPC(

ab1e	43806	strout	Output String

ab3b	43835	outspc	Output Format Character

ab4d	43853	doagin	Handle Bad Data

ab7b	43899	get	Perform [get]

aba5	43941	inputn	Perform [input#]

abbf	43967	input	Perform [input]

abea	44010	bufful	Read Input Buffer

abf9	44025	qinlin	Do Input Prompt

ac06	44038	read	Perform [read]

ac35	44085	rdget	General Purpose Read Routine

acfc	44284	exint	Input Error Messages			TEXT

ad1e	44318	next	Perform [next]

ad61	44385	donext	Check Valid Loop

ad8a	44426	frmnum	Confirm Result

ad9e	44446	frmevl	Evaluate Expression in Text

ae83	44675	eval	Evaluate Single Term

aea8	44712	pival	Constant - pi				DATA

aead	44717	qdot	Continue Expression

aef1	44785	parchk	Expression in Brackets

aef7	44791	chkcls	Confirm Character

aef7	44791	-	-test ')'-

aefa	44794	-	-test '('-

aefd	44797	-	-test comma-

af08	44808	synerr	Output ?SYNTAX Error

af0d	44813	domin	Set up NOT Function

af14	44820	rsvvar	Identify Reserved Variable

af28	44840	isvar	Search for Variable

af48	44872	tisasc	Convert TI to ASCII String

afa7	44967	isfun	Identify Function Type

afb1	44977	strfun	Evaluate String Function

afd1	45009	numfun	Evaluate Numeric Function

afe6	45030	orop	Perform [or], [and]

b016	45078	dorel	Perform <, =, >

b01b	45083	numrel	Numeric Comparison

b02e	45102	strrel	String Comparison

b07e	45182	dim	Perform [dim]

b08b	45195	ptrget	Identify Variable

b0e7	45287	ordvar	Locate Ordinary Variable

b11d	45341	notfns	Create New Variable

b128	45352	notevl	Create Variable

b194	45460	aryget	Allocate Array Pointer Space

b1a5	45477	n32768	Constant 32768 in Flpt			DATA

b1aa	45482	facinx	FAC#1 to Integer in (AC/YR)

b1b2	45490	intidx	Evaluate Text for Integer

b1bf	45503	ayint	FAC#1 to Positive Integer

b1d1	45521	isary	Get Array Parameters

b218	45592	fndary	Find Array

b245	45637	bserr	?BAD SUBSCRIPT/?ILLEGAL QUANTITY

b261	45665	notfdd	Create Array

b30e	45838	inlpn2	Locate Element in Array

b34c	45900	umult	Number of Bytes in Subscript

b37d	45949	fre	Perform [fre]

b391	45969	givayf	Convert Integer in (AC/YR) to Flpt

b39e	45982	pos	Perform [pos]

b3a6	45990	errdir	Confirm Program Mode

b3e1	46049	getfnm	Check Syntax of FN

b3f4	46068	fndoer	Perform [fn]

b465	46181	strd	Perform [str$]

b487	46215	strlit	Set Up String

b4d5	46293	putnw1	Save String Descriptor

b4f4	46324	getspa	Allocate Space for String

b526	46374	garbag	Garbage Collection

b5bd	46525	dvars	Search for Next String

b606	46598	grbpas	Collect a String

b63d	46653	cat	Concatenate Two Strings

b67a	46714	movins	Store String in High RAM

b6a3	46755	frestr	Perform String Housekeeping

b6db	46811	frefac	Clean Descriptor Stack

b6ec	46828	chrd	Perform [chr$]

b700	46848	leftd	Perform [left$]

b72c	46892	rightd	Perform [right$]

b737	46903	midd	Perform [mid$]

b761	46945	pream	Pull sTring Parameters

b77c	46972	len	Perform [len]

b782	46978	len1	Exit String Mode

b78b	46987	asc	Perform [asc]

b79b	47003	gtbytc	Evaluate Text to 1 Byte in XR

b7ad	47021	val	Perform [val]

b7b5	47029	strval	Convert ASCII String to Flpt

b7eb	47083	getnum	Get parameters for POKE/WAIT

b7f7	47095	getadr	Convert FAC#1 to Integer in LINNUM

b80d	47117	peek	Perform [peek]

b824	47140	poke	Perform [poke]

b82d	47149	wait	Perform [wait]

b849	47177	faddh	Add 0.5 to FAC#1

b850	47184	fsub	Perform Subtraction

b862	47202	fadd5	Normalise Addition

b867	47207	fadd	Perform Addition

b947	47431	negfac	2's Complement FAC#1

b97e	47486	overr	Output ?OVERFLOW Error

b983	47491	mulshf	Multiply by Zero Byte

b9bc	47548	fone	Table of Flpt Constants			DATA

b9ea	47594	log	Perform [log]

ba28	47656	fmult	Perform Multiply

ba59	47705	mulply	Multiply by a Byte

ba8c	47756	conupk	Load FAC#2 From Memory

bab7	47799	muldiv	Test Both Accumulators

bad4	47828	mldvex	Overflow / Underflow

bae2	47842	mul10	Multiply FAC#1 by 10

baf9	47865	tenc	Constant 10 in Flpt			DATA

bafe	47870	div10	Divide FAC#1 by 10

bb07	47879	fdiv	Divide FAC#2 by Flpt at (AC/YR)

bb0f	47887	fdivt	Divide FAC#2 by FAC#1

bba2	48034	movfm	Load FAC#1 From Memory

bbc7	48071	mov2f	Store FAC#1 in Memory

bbfc	48124	movfa	Copy FAC#2 into FAC#1

bc0c	48140	movaf	Copy FAC#1 into FAC#2

bc1b	48155	round	Round FAC#1

bc2b	48171	sign	Check Sign of FAC#1

bc39	48185	sgn	Perform [sgn]

bc58	48216	abs	Perform [abs]

bc5b	48219	fcomp	Compare FAC#1 With Memory

bc9b	48283	qint	Convert FAC#1 to Integer

bccc	48332	int	Perform [int]

bcf3	48371	fin	Convert ASCII String to a Number in FAC#1

bdb3	48563	n0999	String Conversion Constants		DATA

bdc2	48578	inprt	Output 'IN' and Line Number

bddd	48605	fout	Convert FAC#1 to ASCII String

be68	48744	foutim	Convert TI to String

bf11	48913	fhalf	Table of Constants			DATA

bf71	49009	sqr	Perform [sqr]

bf7b	49019	fpwrt	Perform power ($)

bfb4	49076	negop	Negate FAC#1

bfbf	49087	logeb2	Table of Constants			DATA

bfed	49133	exp	Perform [exp]

C000-CFFF   49152   4 KB RAM.

D000-DFFF   53248   Input/Output Devices and Colour RAM or

4 KB RAM or Character ROM.

D000-D02E   53248   6566 Video Interface Chip, VIC II.

D000-D02E	53248-54271	MOS 6566 VIDEO INTERFACE CONTROLLER (VIC)

D000		53248		Sprite O X Pos

D001		53249		Sprite O Y Pos

D002		53250		Sprite 1 X Pos

D003		53251		Sprite 1 Y Pos

D004		53252		Sprite 2 X Pos

D005		53253		Sprite 2 Y Pos

D006		53254		Sprite 3 X Pos

D007		53255		Sprite 3 Y Pos

D008		53256		Sprite 4 X Pos

D009		53257		Sprite 4 Y Pos

D00A		53258		Sprite 5 X Pos

D00B		53259		Sprite 5 Y Pos

D00C		53260		Sprite 6 X Pos

D00D		53261		Sprite 6 Y Pos

D00E		53262		Sprite 7 X Pos

D00F		53263		Sprite 7 Y Pos

D010		53264		Sprites 0-7 X Pos (msb of X coord.)

D011		53265		VIC Control Register

7	Raster Compare: (Bit 8)	See 53266

6	Extended Color Text Mode 1 = Enable

5	Bit Map Mode. 1 = Enable

4	Blank Screen to Border Color: O = Blank

3	Select 24/25 Row Text Display: 1 = 25 Rows

2-0	Smooth Scroll to Y Dot-Position (0-7)

D012	53266			Read Raster / Write Raster Value for Compare IRQ

D013	53267			Light-Pen Latch X Pos

D014	53268			Light-Pen Latch Y Pos

D015	53269			Sprite display Enable: 1 = Enable

D016	53270			VIC Control Register

7-6	Unused

5	ALWAYS SET THIS BIT TO 0 !

4	Multi-Color Mode: 1 = Enable (Text or Bit-Map)

3	Select 38/40 Column Text Display: 1 = 40 Cols

2-0	Smooth Scroll to X Pos

D017	53271			Sprites O-7 Expand 2x Vertical (Y)

D018	53272			VIC Memory Control Register

7-4	Video Matrix Base Address (inside VIC)

3-1	Character Dot-Data Base	Address (inside VIC)

0	Select upper/lower Character Set

D019	53273			VIC Interrupt Flag Register (Bit = 1: IRQ Occurred)

7	Set on Any Enabled VIC IRQ Condition

3	Light-Pen Triggered IRQ Flag

2	Sprite to Sprite Collision IRQ Flag

1	Sprite to Background Collision IRQ Flag

0	Raster Compare IRQ Flag

D01A	53274			IRQ Mask Register: 1 = Interrupt Enabled

D01B	53275			Sprite to Background Display Priority: 1 = Sprite

D01C	53276			Sprites O-7 Multi-Color Mode Select: 1 = M.C.M.

D01D	53277			Sprites 0-7 Expand 2x Horizontal (X)

D01E	53278			Sprite to Sprite Collision Detect

D01F	53279			Sprite to Background Collision Detect

D020	53280			Border Color

D021	53281			Background Color O

D022	53282			Background Color 1

D023	53283			Background Color 2

D024	53284			Background Color 3

D025	53285			Sprite Multi-Color Register 0

D026	53286			Sprite Multi-Color Register 1

D027	53287			Sprite O Color

D028	53288			Sprite 1 Color

D029	53289			Sprite 2 Color

D02A	53290			Sprite 3 Color

D02B	53291			Sprite 4 Color

D02C	53292			Sprite 5 Color

D02D	53293			Sprite 6 Color

D02E	53294			Sprite 7 Color

D400-D41C   54272   6581 Sound Interface Device, SID.

D400-D7FF	54272-55295	MOS 6581 SOUND INTERFACE DEVICE (SID)

D400	54272			Voice 1: Frequency Control - Low-Byte

D401	54273			Voice 1: Frequency Control - High-Byte

D402	54274			Voice 1: Pulse Waveform	Width - Low-Byte

D403	54275		7-4	Unused

3-0	Voice 1: Pulse Waveform Width - High-Nybble

D404	54276			Voice 1: Control Register

7	Select Random Noise Waveform, 1 = On

6	Select Pulse Waveform, 1 = On

5	Select Sawtooth Waveform, 1 = On

4	Select Triangle Waveform, 1 = On

3	Test Bit: 1 = Disable Oscillator 1

2	Ring Modulate Osc. 1 with Osc. 3 Output, 1 = On

1	Synchronize Osc. 1 with Osc. 3 Frequency, 1 = On

0	Gate Bit: 1 = Start Att/Dec/Sus, 0 = Start Release

D405	54277			Envelope Generator 1: Attack / Decay Cycle Control

7-4	Select Attack Cycle Duration: O-15

3-0	Select Decay Cycle Duration: 0-15

D406	54278			Envelope Generator 1: Sustain / Release Cycle Control

7-4	Select Sustain Cycle Duration: O-15

3-0	Select Release Cycle Duration: O-15

D407	54279			Voice 2: Frequency Control - Low-Byte

D408	54280			Voice 2: Frequency Control - High-Byte

D409	54281			Voice 2: Pulse Waveform Width - Low-Byte

D40A	54282		7-4	Unused

3-0	Voice 2: Pulse Waveform Width - High-Nybble

D40B	54283			Voice 2: Control Register

7	Select Random Noise Waveform, 1 = On

6	Select Pulse Waveform, 1 = On

5	Select Sawtooth Waveform, 1 = On

4	Select Triangle Waveform, 1 = On

3	Test Bit: 1 = Disable Oscillator 1

2	Ring Modulate Osc. 2 with Osc. 1 Output, 1 = On

1	Synchronize Osc. 2 with Osc. 1 Frequency, 1 = On

0	Gate Bit: 1 = Start Att/Dec/Sus, 0 = Start Release

D40C	54284			Envelope Generator 2: Attack / Decay Cycle Control

7-4	Select Attack Cycle Duration: O-15

3-0	Select Decay Cycle Duration: 0-15

D40D	54285			Envelope Generator 2: Sustain / Release Cycle Control

7-4	Select Sustain Cycle Duration: O-15

3-0	Select Release Cycle Duration: O-15

D40E	54286			Voice 3: Frequency Control - Low-Byte

D40F	54287			Voice 3: Frequency Control - High-Byte

D410	54288			Voice 3: Pulse Waveform Width - Low-Byte

D411	54289		7-4	Unused

3-0	Voice 3: Pulse Waveform Width - High-Nybble

D412	54290			Voice 3: Control Register

7	Select Random Noise Waveform, 1 = On

6	Select Pulse Waveform, 1 = On

5	Select Sawtooth Waveform, 1 = On

4	Select Triangle Waveform, 1 = On

3	Test Bit: 1 = Disable Oscillator 1

2	Ring Modulate Osc. 3 with Osc. 2 Output, 1 = On

1	Synchronize Osc. 3 with Osc. 2 Frequency, 1 = On

0	Gate Bit: 1 = Start Att/Dec/Sus, 0 = Start Release

D413	54291		Envelope Generator 3: Attac/Decay Cycle Control

7-4	Select Attack Cycle Duration: O-15

3-0	Select Decay Cycle Duration: 0-15

D414	54285			Envelope Generator 3: Sustain / Release Cycle Control

7-4	Select Sustain Cycle Duration: O-15

3-0	Select Release Cycle Duration: O-15

D415	54293			Filter Cutoff Frequency: Low-Nybble (Bits 2-O)

D416	54294			Filter Cutoff Frequency: High-Byte

D417	54295			Filter Resonance Control / Voice Input Control

7-4	Select Filter Resonance: 0-15

3	Filter External Input: 1 = Yes, 0 = No

2	Filter Voice 3 Output: 1 = Yes, 0 = No

Filter Voice 2 Output: 1 = Yes, 0 = No

0	Filter Voice 1 Output: 1 = Yes, 0 = No

D418	54296			Select Filter Mode and Volume

7	Cut-Off Voice 3 Output: 1 = Off, O = On

6	Select Filter High-Pass Mode: 1 = On

5	Select Filter Band-Pass Mode: 1 = On

4	Select Filter Low-Pass Mode: 1 = On

3-0	Select Output Volume: 0-15

D419	54297			Analog/Digital Converter: Game Paddle 1 (O-255)

D41A	54298			Analog/Digital Converter Game Paddle 2 (O-255)

D41B	54299			Oscillator 3 Random Number Generator

D41C	54230			Envelope Generator 3 Output

D500-D7FF   54528   SID Images.

D800-DBE7   55296   Colour RAM (Nybbles = 4 Bit RAM, LSB).

DBE8-DBFF   56296   Unused Nybbles.

DC00-DC0F   56320   6526 Complex Interface Adaptor, CIA.

DC00	56320			Data Port A (Keyboard, Joystick, Paddles, Light-Pen)

7-0	Write Keyboard Column Values for Keyboard Scan

7-6	Read Paddles on Port A / B (01 = Port A, 10 = Port B)

4	Joystick A Fire Button: 1 = Fire

3-2	Paddle Fire Buttons

3-0	Joystick A Direction (0-15)

DC01	56321			Data Port B (Keyboard, Joystick, Paddles): Game Port 1

7-0	Read Keyboard Row Values for Keyboard Scan

7	Timer B Toggle/Pulse Output

6	Timer A: Toggle/Pulse Output

4	Joystick 1 Fire Button: 1 = Fire

3-2	Paddle Fire Buttons

3-0	Joystick 1 Direction

DC02	56322			Data Direction Register - Port A (56320)

DC03	56323			Data Direction Register - Port B (56321)

DC04	56324			Timer A: Low-Byte

DC05	56325			Timer A: High-Byte

DC06	56326			Timer B: Low-Byte

DC07	56327			Timer B: High-Byte

DC08	56328			Time-of-Day Clock: 1/10 Seconds

DC09	56329			Time-of-Day Clock: Seconds

DC0A	56330			Time-of-Day Clock: Minutes

DC0B	56331			Time-of-Day Clock: Hours + AM/PM Flag (Bit 7)

DC0C	56332			Synchronous Serial I/O Data Buffer

DC0D	56333			CIA Interrupt Control Register (Read IRQs/Write Mask)

7	IRQ Flag (1 = IRQ Occurred) / Set-Clear Flag

4	FLAG1 IRQ (Cassette Read / Serial Bus SRQ Input)

3	Serial Port Interrupt

2	Time-of-Day Clock Alarm Interrupt

1	Timer B Interrupt

0	Timer A Interrupt

DC0E	56334			CIA Control Register A

7	Time-of-Day Clock Frequency: 1 = 50 Hz, 0 = 60 Hz

6	Serial Port I/O Mode Output, 0 = Input

5	Timer A Counts: 1 = CNT Signals, 0 = System 02 Clock

4	Force Load Timer A: 1 = Yes

3	Timer A Run Mode: 1 = One-Shot, 0 = Continuous

2	Timer A Output Mode to PB6: 1 = Toggle, 0 = Pulse

1	Timer A Output on PB6: 1 = Yes, 0 = No

0	Start/Stop Timer A: 1 = Start, 0 = Stop

DC0F	56335			CIA Control Register B

7	Set Alarm/TOD-Clock: 1 = Alarm, 0 = Clock

6-5	Timer B Mode Select:

00 = Count System 02 Clock Pulses

01 = Count Positive CNT Transitions

10 = Count Timer A Underflow Pulses

11 = Count Timer A Underflows While CNT Positive

4-0	Same as CIA Control Reg. A - for Timer B

DC00-DCFF	56320-56575	MOS 6526 Complex Interface Adapter (CIA) #1

DD00-DDFF	56576-56831	MOS 6526 Complex Interface Adapter (CIA) #2

DD00	56576			Data Port A (Serial Bus, RS-232, VIC Memory Control)

7	Serial Bus Data Input

6	Serial Bus Clock Pulse Input

5	Serial Bus Data Output

4	Serial Bus Clock Pulse Output

3	Serial Bus ATN Signal Output

2	RS-232 Data Output (User Port)

1-O	VIC Chip System Memory Bank Select (Default = 11)

DD01	56577		Data Port B (User Port, RS-232)

7	User / RS-232 Data Set Ready

6	User / RS-232 Clear to Send

5	User

4	User / RS-232 Carrier Detect

3	User / RS-232 Ring Indicator

2	User / RS-232 Data Terminal Ready

1	User / RS-232 Request to Send

0	User / RS-232 Received Data

DD02	56578			Data Direction Register - Port A

DD03	56579			Data Direction Register - Port B

DD04	56580			Timer A: Low-Byte

DD05	56581			Timer A: High-Byte

DD06	56582			Timer B: Low-Byte

DD07	56583			Timer B: High-Byte

DD08	56584			Time-of-Day Clock: 1/10 Seconds

DD09	56585			Time-of-Day Clock: Seconds

DD0A	56586			Time-of-Day Clock: Minutes

DD0B	56587			Time-of-Day Clock: Hours + AM/PM Flag (Bit 7)

DD0C	56588			Synchronous Serial I/O Data Buffer

DD0D	56589			CIA Interrupt Control Register (Read NMls/Write Mask)

7	NMI Flag (1 = NMI Occurred) / Set-Clear Flag

4	FLAG1 NMI (User/RS-232 Received Data Input)

3	Serial Port Interrupt

1	Timer B Interrupt

0	Timer A Interrupt

DD0E	56590			CIA Control Register A

7	Time-of-Day Clock Frequency: 1 = 50 Hz, 0 = 60 Hz

6	Serial Port I/O Mode Output, 0 = Input

5	Timer A Counts: 1 = CNT Signals, 0 = System 02 Clock

4	Force Load Timer A: 1 = Yes

3	Timer A Run Mode: 1 = One-Shot, 0 = Continuous

2	Timer A Output Mode to PB6: 1 = Toggle, 0 = Pulse

1	Timer A Output on PB6: 1 = Yes, 0 = No

0	Start/Stop Timer A: 1 = Start, 0 = Stop

DD0F	56591			CIA Control Register B

7	Set Alarm/TOD-Clock: 1 = Alarm, 0 = Clock

6-5	Timer B Mode Select:

00 = Count System 02 Clock Pulses

01 = Count Positive CNT Transitions

10 = Count Timer A Underflow Pulses

11 = Count Timer A Underflows While CNT Positive

4-0	Same as CIA Control Reg. A - for Timer B

DEOO-DEFF	56832-57087	Reserved for Future I/O Expansion

DFOO-DFFF	57088-57343	Reserved for Future I/O Expansion

E000-FFFF   57344   BASIC (Part)/Kernal ROM or 8 KB RAM.

E000-E4FF   57344   BASIC ROM (Part) or RAM.

e000	57344	(exp continues)	EXP continued From BASIC ROM

e043	57411	polyx	Series Evaluation

e08d	57485	rmulc	Constants for RND			DATA

e097	57495	rnd	Perform [rnd]

e0f9	57593	bioerr	Handle I/O Error in BASIC

e10c	57612	bchout	Output Character

e112	57618	bchin	Input Character

e118	57624	bckout	Set Up For Output

e11e	57630	bckin	Set Up For Input

e124	57636	bgetin	Get One Character

e12a	57642	sys	Perform [sys]

e156	57686	savet	Perform [save]

e165	57701	verfyt	Perform [verify / load]

e1be	57790	opent	Perform [open]

e1c7	57799	closet	Perform [close]

e1d4	57812	slpara	Get Parameters For LOAD/SAVE

e200	57856	combyt	Get Next One Byte Parameter

e206	57862	deflt	Check Default Parameters

e20e	57870	cmmerr	Check For Comma

e219	57881	ocpara	Get Parameters For OPEN/CLOSE

e264	57956	cos	Perform [cos]

e26b	57963	sin	Perform [sin]

e2b4	58036	tan	Perform [tan]

e2e0	58080	pi2	Table of Trig Constants			DATA

;e2e0	1.570796327	pi/2

;e2e5	6.28318531	pi*2

;e2ea	0.25

;e2ef	#05	(counter)

;e2f0	-14.3813907

;e2f5	42.0077971

;e2fa	-76.7041703

;e2ff	81.6052237

;e304	-41.3417021

;e309	6.28318531

e30e	58126	atn	Perform [atn]

e33e	58174	atncon	Table of ATN Constants			DATA

;e33e	#0b	(counter)

;e3ef	-0.000684793912

;e344	 0.00485094216

;e349	-0.161117018

;e34e	 0.034209638

;e353	-0.0542791328

;e358	 0.0724571965

;e35d	-0.0898023954

;e362	 0.110932413

;e367	-0.142839808

;e36c	 0.19999912

;e371	-0.333333316

;e376	 1.00

e37b	58235	bassft	BASIC Warm Start [RUNSTOP-RESTORE]

e394	58260	init	BASIC Cold Start

e3a2	58274	initat	CHRGET For Zero-page

e3ba	58298	rndsed	RND Seed For zero-page			DATA

;e3b2	0.811635157

e3bf	58303	initcz	Initialize BASIC RAM

e422	58402	initms	Output Power-Up Message

e447	58439	bvtrs	Table of BASIC Vectors (for 0300)	WORD

e453	58451	initv	Initialize Vectors

e45f	58463	words	Power-Up Message			DATA

e4ad	58541	-	Patch for BASIC Call to CHKOUT

e4b7	58551	-	Unused Bytes For Future Patches		EMPTY

e4da	58586	-	Reset Character Colour

e4e0	58592	-	Pause After Finding Tape File

e4ec	58604	-	RS-232 Timing Table -- PAL		DATA

E500-FFFF   58624   Kernal ROM or RAM.

e500	58624	iobase	Get I/O Address

e505	58629	screen	Get Screen Size

e50a	58634	plot	Put / Get Row And Column

e518	58648	cint1	Initialize I/O

e544	58692	-	Clear Screen

e566	58726	-	Home Cursor

e56c	58732	-	Set Screen Pointers

e59a	58778	-	Set I/O Defaults (Unused Entry)

e5a0	58784	-	Set I/O Defaults

e5b4	58804	lp2	Get Character From Keyboard Buffer

e5ca	58826	-	Input From Keyboard

e632	58930	-	Input From Screen or Keyboard

e684	59012	-	Quotes Test

e691	59025	-	Set Up Screen Print

e6b6	59062	-	Advance Cursor

e6ed	59117	-	Retreat Cursor

e701	59137	-	Back on to Previous Line

e716	59158	-	Output to Screen

e72a	59178	-	-unshifted characters-

e7d4	59348	-	-shifted characters-

e87c	59516	-	Go to Next Line

e891	59537	-	Output <CR>

e8a1	59553	-	Check Line Decrement

e8b3	59571	-	Check Line Increment

e8cb	59595	-	Set Colour Code

e8da	59610	-	Colour Code Table

e8ea	59626	-	Scroll Screen

e965	59749	-	Open A Space On The Screen

e9c8	59848	-	Move A Screen Line

e9e0	59872	-	Syncronise Colour Transfer

e9f0	59888	-	Set Start of Line

e9ff	59903	-	Clear Screen Line

ea13	59923	-	Print To Screen

ea24	59940	-	Syncronise Colour Pointer

ea31	59953	-	Main IRQ Entry Point

ea87	60039	scnkey	Scan Keyboard

eadd	60125	-	Process Key Image

eb79	60281	-	Pointers to Keyboard decoding tables	WORD

eb81	60289	-	Keyboard 1 -- unshifted			DATA

ebc2	60354	-	Keyboard 2 -- Shifted			DATA

ec03	60419	-	Keyboard 3 -- Commodore			DATA

ec44	60484	-	Graphics/Text Control

ec78	60536	-	Keyboard 4 -- Control			DATA

ecb9	60601	-	Video Chip Setup Table			DATA

ece7	60647	-	Shift-Run Equivalent

ecf0	60656	-	Low Byte Screen Line Addresses		DATA

ed09	60681	talk	Send TALK Command on Serial Bus

ed0c	60684	listn	Send LISTEN Command on Serial Bus

ed40	60736	-	Send Data On Serial Bus

edad	60845	-	Flag Errors

edad	60845	-	Status #80 - device not present

edb0	60848	-	Status #03 - write timeout

edb9	60857	second	Send LISTEN Secondary Address

edbe	60862	-	Clear ATN

edc7	60871	tksa	Send TALK Secondary Address

edcc	60876	-	Wait For Clock

eddd	60893	ciout	Send Serial Deferred

edef	60911	untlk	Send UNTALK / UNLISTEN

ee13	60947	acptr	Receive From Serial Bus

ee85	61061	-	Serial Clock On

ee8e	61070	-	Serial Clock Off

ee97	61079	-	Serial Output 1

eea0	61088	-	Serial Output 0

eea9	61097	-	Get Serial Data And Clock In

eeb3	61107	-	Delay 1 ms

eebb	61115	-	RS-232 Send

ef06	61190	-	Send New RS-232 Byte

ef2e	61230	-	'No DSR' / 'No CTS' Error

ef39	61241	-	Disable Timer

ef4a	61258	-	Compute Bit Count

ef59	61273	-	RS-232 Receive

ef7e	61310	-	Set Up To Receive

ef90	61328	-	Process RS-232 Byte

efe1	61409	-	Submit to RS-232

f00d	61453	-	No DSR (Data Set Ready) Error

f017	61463	-	Send to RS-232 Buffer

f04d	61517	-	Input From RS-232

f086	61574	-	Get From RS-232

f0a4	61604	-	Serial Bus Idle

f0bd	61629	-	Table of Kernal I/O Messages		DATA

f12b	61739	-	Print Message if Direct

f12f	61743	-	Print Message

f13e	61758	getin	Get a byte

f157	61783	chrin	Input a byte

f199	61849	-	Get From Tape / Serial / RS-232

f1ca	61898	chrout	Output One Character

f20e	61966	chkin	Set Input Device

f250	62032	chkout	Set Output Device

f291	62097	close	Close File

f30f	62223	-	Find File

f31f	62239	-	Set File values

f32f	62255	clall	Abort All Files

f333	62259	clrchn	Restore Default I/O

f34a	62282	open	Open File

f3d5	62421	-	Send Secondary Address

f409	62473	-	Open RS-232

f49e	62622	load	Load RAM

f4b8	62648	-	Load File From Serial Bus

f533	62771	-	Load File From Tape

f5af	62927	-	Print "SEARCHING"

f5c1	62913	-	Print Filename

f5d2	62930	-	Print "LOADING / VERIFYING"

f5dd	62941	save	Save RAM

f5fa	62970	-	Save to Serial Bus

f659	63065	-	Save to Tape

f68f	63119	-	Print "SAVING"

f69b	63131	udtim	Bump Clock

f6dd	63197	rdtim	Get Time

f6e4	63204	settim	Set Time

f6ed	63213	stop	Check STOP Key

f6fb	63227	-	Output I/O Error Messages

f6fb	63227	-	'too many files'

f6fe	63230	-	'file open'

f701	63233	-	'file not open'

f704	63236	-	'file not found'

f707	63239	-	'device not present'

f70a	63242	-	'not input file'

f70d	63245	-	'not output file'

f710	63248	-	'missing filename'

f713	63251	-	'illegal device number'

f72d	63277	-	Find Any Tape Header

f76a	63338	-	Write Tape Header

f7d0	63440	-	Get Buffer Address

f7d7	63447	-	Set Buffer Stat / End Pointers

f7ea	63466	-	Find Specific Tape Header

f80d	63501	-	Bump Tape Pointer

f817	63511	-	Print "PRESS PLAY ON TAPE"

f82e	63534	-	Check Tape Status

f838	63544	-	Print "PRESS RECORD..."

f841	63553	-	Initiate Tape Read

f864	63588	-	Initiate Tape Write

f875	63605	-	Common Tape Code

f8d0	63696	-	Check Tape Stop

f8e2	63714	-	Set Read Timing

f92c	63788	-	Read Tape Bits

fa60	64096	-	Store Tape Characters

fb8e	64398	-	Reset Tape Pointer

fb97	64407	-	New Character Setup

fba6	64422	-	Send Tone to Tape

fbc8	64456	-	Write Data to Tape

fbcd	64461	-	IRQ Entry Point

fc57	64599	-	Write Tape Leader

fc93	64659	-	Restore Normal IRQ

fcb8	64696	-	Set IRQ Vector

fcca	64714	-	Kill Tape Motor

fcd1	64721	-	Check Read / Write Pointer

fcdb	64731	-	Bump Read / Write Pointer

fce2	64738	-	Power-Up RESET Entry

fd02	64770	-	Check For 8-ROM

fd12	64786	-	8-ROM Mask '80CBM'			DATA

fd15	64789	restor	Restore Kernal Vectors (at 0314)

fd1a	64794	vector	Change Vectors For User

fd30	64816	-	Kernal Reset Vectors			WORD

fd50	64848	ramtas	Initialise System Constants

fd9b	64923	-	IRQ Vectors For Tape I/O		WORD

fda3	64931	ioinit	Initialise I/O

fddd	64989	-	Enable Timer

fdf9	65017	setnam	Set Filename

fe00	65024	setlfs	Set Logical File Parameters

fe07	65031	readst	Get I/O Status Word

fe18	65048	setmsg	Control OS Messages

fe21	65057	settmo	Set IEEE Timeout

fe25	65061	memtop	Read / Set Top of Memory

fe34	65076	membot	Read / Set Bottom of Memory

fe43	65091	-	NMI Transfer Entry

fe66	65126	-	Warm Start Basic [BRK]

febc	65212	-	Exit Interrupt

fec2	65218	-	RS-232 Timing Table - NTSC	DATA

fed6	65238	-	NMI RS-232 In

ff07	65287	-	NMI RS-232 Out

ff43	65347	-	Fake IRQ Entry

ff48	65352	-	IRQ Entry

ff5b	65371	cint	Initialize screen editor

ff80	65408	-	Kernal Version Number [03]	DATA

APPENDIX B

----------

; ---<FROM FILE C64rom.lib>---

;

;	Commodore 64 ROM Memory Map

;

; BASIC interpreter ROM ($A000 - $BFFF)

;

; label	  address	type	comments

restart	= $a000

stmdsp	= $a00c

fundsp	= $a052

optab	= $a080

reslst	= $a09e

msclst	= $a129

oplist	= $a140

funlst	= $a14d

errtab	= $a19e

errptr	= $a328

okk	= $a364

fndfor	= $a38a

bltu	= $a3b8

getstk	= $a3fb

reason	= $a408

omerr	= $a435

error	= $a437

errfin	= $a469

ready	= $a474

main	= $a480

main1	= $a49c

inslin	= $a4a2

linkprg	= $a533

inlin	= $a560

crunch	= $a579

fndlin	= $a613

scrtch	= $a642

clear	= $a65e

stxpt	= $a68e

list	= $a69c

qplop	= $a717

for	= $a742

newstt	= $a7ae

ckeol	= $a7c4

gone	= $a7e1

gone3	= $a7ed

restor	= $a81d

stop	= $a82c

cont	= $a857

run	= $a871

gosub	= $a883

goto	= $a8a0

return	= $a8d2

data	= $a8f8

datan	= $a906

if	= $a928

rem	= $a93b

ongoto	= $a94b

linget	= $a96b

let	= $a9a5

putint	= $a9c4

ptflpt	= $a9d6

putstr	= $a9d9

puttim	= $a9e3

getspt	= $aa2c

printn	= $aa80

cmd	= $aa86

strdon	= $aa9a

print	= $aaa0

varop	= $aab8

crdo	= $aad7

comprt	= $aae8

strout	= $ab1e

outspc	= $ab3b

doagin	= $ab4d

get	= $ab7b

inputn	= $aba5

input	= $abbf

bufful	= $abea

qinlin	= $abf9

read	= $ac06

rdget	= $ac35

exint	= $acfc

next	= $ad1e

donext	= $ad61

frmnum	= $ad8a

frmevl	= $ad9e

eval	= $ae83

pival	= $aea8

qdot	= $aead

parchk	= $aef1

chkcls	= $aef7

synerr	= $af08

domin	= $af0d

rsvvar	= $af14

isvar	= $af28

tisasc	= $af48

isfun	= $afa7

strfun	= $afb1

numfun	= $afd1

orop	= $afe6

dorel	= $b016

numrel	= $b01b

strrel	= $b02e

dim	= $b07e

ptrget	= $b08b

ordvar	= $b0e7

isletc	= $b113

notfns	= $b11d

notevl	= $b128

aryget	= $b194

n32768	= $b1a5		data

facinx	= $b1aa

intidx	= $b1b2

ayint	= $b1bf

isary	= $b1d1

fndary	= $b218

bserr	= $b245

notfdd	= $b261

inlpn2	= $b30e

umult	= $b34c

fre	= $b37d

givayf	= $b391

pos	= $b39e

errdir	= $b3a6

def	= $b3b3

getfnm	= $b3e1

fndoer	= $b3f4

strd	= $b465

strlit	= $b487

putnw1	= $b4d5

getspa	= $b4f4

garbag	= $b526

dvars	= $b5bd

grbpas	= $b606

cat	= $b63d

movins	= $b67a

frestr	= $b6a3

frefac	= $b6db

chrd	= $b6ec

leftd	= $b700

rightd	= $b72c

midd	= $b737

pream	= $b761

len	= $b77c

len1	= $b782

asc	= $b78b

gtbytc	= $b79b

val	= $b7ad

strval	= $b7b5

getnum	= $b7eb

getadr	= $b7f7

peek	= $b80d

poke	= $b824

wait	= $b82d

faddh	= $b849

fsub	= $b850

fadd5	= $b862

fadd	= $b867

negfac	= $b947

overr	= $b97e

mulshf	= $b983

fone	= $b9bc		data

log	= $b9ea

fmult	= $ba28

mulply	= $ba59

conupk	= $ba8c

muldiv	= $bab7

mldvex	= $bad4

mul10	= $bae2

tenc	= $baf9		data

div10	= $bafe

fdiv	= $bb07

fdivt	= $bb0f

movfm	= $bba2

mov2f	= $bbc7

movfa	= $bbfc

movaf	= $bc0c

round	= $bc1b

sign	= $bc2b

sgn	= $bc39

abs	= $bc58

fcomp	= $bc5b

qint	= $bc9b

int	= $bccc

fin	= $bcf3

n0999	= $bdb3		data

inprt	= $bdc2

fout	= $bddd

foutim	= $be68

fhalf	= $bf11		data

sqr	= $bf71

fpwrt	= $bf7b

negop	= $bfb4

logeb2	= $bfbf		data

exp	= $bfed

;

;

;	C64 KERNEL ROM

;

(exp	= $e000

polyx	= $e043

rmulc	= $e08d		data

rnd	= $e097

bioerr	= $e0f9

bchout	= $e10c

bchin	= $e112

bckout	= $e118

bckin	= $e11e

bgetin	= $e124

sys	= $e12a

savet	= $e156

verfyt	= $e165

opent	= $e1be

closet	= $e1c7

slpara	= $e1d4

combyt	= $e200

deflt	= $e206

cmmerr	= $e20e

ocpara	= $e219

cos	= $e264

sin	= $e26b

tan	= $e2b4

pi2	= $e2e0		data

atn	= $e30e

atncon	= $e33e		data

bassft	= $e37b

init	= $e394

initat	= $e3a2

rndsed	= $e3ba

initcz	= $e3bf

initms	= $e422

bvtrs	= $e447		data

initv	= $e453

words	= $e45f

-	= $e4ad

-	= $e4b7		illegal

-	= $e4da

-	= $e4e0

-	= $e4ec		data

iobase	= $e500

screen	= $e505

plot	= $e50a

cint1	= $e518

-	= $e544

-	= $e566

-	= $e56c

= ;

-	= $e59a

lp2	= $e5b4

-	= $e5ca

-	= $e632

-	= $e684

-	= $e691

-	= $e6b6

-	= $e6ed

-	= $e701

-	= $e716

-	= $e87c

-	= $e891

-	= $e8a1

-	= $eacb

-	= $e8da

-	= $e8ea

-	= $e965

-	= $e9c8

-	= $e9e0

-	= $e9f0

-	= $e9ff

-	= $ea13

-	= $ea24

-	= $ea31

scnkey	= $ea87

-	= $eadd		data

-	= $eb79		data

-	= $eb81		data

-	= $ebc2		data

-	= $ec03

-	= $ec44		data

-	= $ec78		data

-	= $ecb9

-	= $ece7		data

-	= $ecf0

talk	= $ed09

-	= $ed40

-	= $edad

second	= $edb9

-	= $edbe

tksa	= $edc7

-	= $edcc

ciout	= $eddd

untlk	= $edef

acptr	= $ee13

-	= $ee85

-	= $ee8e

-	= $ee97

-	= $eea0

-	= $eea9

-	= $eeb3

-	= $eebb

-	= $ef06

-	= $ef2e

-	= $ef39

-	= $ef4a

-	= $ef59

-	= $ef7e

-	= $ef90

-	= $efe1

-	= $f00d

-	= $f017

-	= $f04d

-	= $f086

-	= $f0a4

-	= $f0bd

-	= $f128

getin	= $f13e

chrin	= $f157

-	= $f199

chrout	= $f1ca

chkin	= $f20e

chkout	= $f250

close	= $f291

-	= $f30f

-	= $f31f

clall	= $f32f

clrchn	= $f333

open	= $f34a

-	= $f3d5

-	= $f409

load	= $f49e

;

;--------------

;

save	= $f5dd

udtim	= $f69b

rdtim	= $f6dd

settim	= $f6e4

stop	= $f6ed

restor	= $fd15

vector	= $fd1a

ramtas	= $fd50

ioinit	= $fda3

setnam	= $fdf9

setlfs	= $fe00

readst	= $fe07

setmsg	= $fe18

settmo	= $fe21

memtop	= $fe25

membot	= $fe34

cint	= $fe58

APPENDIX C

----------

;

;

; C64 KERNEL call addresses

;

acptr	= $ffa5

chkin	= $ffc6

chkout	= $ffc9

chrin	= $ffcf

chrout	= $ffd2

ciout	= $ffa8

cint	= $ff81

clall	= $ffe7

close	= $ffc3

clrchn	= $ffcc

getin	= $ffe4

iobase	= $fff3

ioinit	= $ff84

listen	= $ffb1

load	= $ffd5

membot	= $ff9c

memtop	= $ff99

open	= $ffc0

plot	= $fff0

ramtas	= $ff87

rdtim	= $ffde

readst	= $ffb7

restor	= $ff8a

save	= $ffd8

scnkey	= $ff9f

screen	= $ffed

second	= $ff93

setlfs	= $ffba

setmsg	= $ff90

setnam	= $ffbd

settim	= $ffdb

settmo	= $ffa2

stop	= $ffe1

talk	= $ffb4

tksa	= $ff96

udtim	= $ffea

unlsn	= $ffae

untlk	= $ffab

vector	= $ff8d

;

APPENDIX D

----------

OPCODES::

--------------

REPRODUCED FROM C=HACKING MAGAZINE..

6502 Opcodes and Quasi-Opcodes.

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

The following table lists all of the available opcodes on the 65xx line of

micro-processors (such as the 6510 on the C=64 and the 8502 on the C=128)

-----------------------------------------------------------------------------

Std Mnemonic Hex Value Description                Addressing Mode  Bytes/Time

*   BRK      $00       Stack <- PC, PC <- ($fffe) (Immediate)      1/7

*   ORA      $01       A <- (A) V M               (Ind,X)          6/2

JAM      $02       [locks up machine]         (Implied)        1/-

SLO      $03       M <- (M >> 1) + A + C      (Ind,X)          2/8

NOP      $04       [no operation]             (Z-Page)         2/3

*   ORA      $05       A <- (A) V M               (Z-Page)         2/3

*   ASL      $06       C <- A7, A <- (A) << 1     (Z-Page)         2/5

SLO      $07       M <- (M >> 1) + A + C      (Z-Page)         2/5

*   PHP      $08       Stack <- (P)               (Implied)        1/3

*   ORA      $09       A <- (A) V M               (Immediate)      2/2

*   ASL      $0A       C <- A7, A <- (A) << 1     (Accumalator)    1/2

ANC      $0B       A <- A /\ M, C=~A7         (Immediate)      1/2

NOP      $0C       [no operation]             (Absolute)       3/4

*   ORA      $0D       A <- (A) V M               (Absolute)       3/4

*   ASL      $0E       C <- A7, A <- (A) << 1     (Absolute)       3/6

SLO      $0F       M <- (M >> 1) + A + C      (Absolute)       3/6

*   BPL      $10       if N=0, PC = PC + offset   (Relative)       2/2'2

*   ORA      $11       A <- (A) V M               ((Ind),Y)        2/5'1

JAM      $12       [locks up machine]         (Implied)        1/-

SLO      $13       M <- (M >. 1) + A + C      ((Ind),Y)        2/8'5

NOP      $14       [no operation]             (Z-Page,X)       2/4

*   ORA      $15       A <- (A) V M               (Z-Page,X)       2/4

*   ASL      $16       C <- A7, A <- (A) << 1     (Z-Page,X)       2/6

SLO      $17       M <- (M >> 1) + A + C      (Z-Page,X)       2/6

*   CLC      $18       C <- 0                     (Implied)        1/2

*   ORA      $19       A <- (A) V M               (Absolute,Y)     3/4'1

NOP      $1A       [no operation]             (Implied)        1/2

SLO      $1B       M <- (M >> 1) + A + C      (Absolute,Y)     3/7

NOP      $1C       [no operation]             (Absolute,X)     2/4'1

*   ORA      $1D       A <- (A) V M               (Absolute,X)     3/4'1

*   ASL      $1E       C <- A7, A <- (A) << 1     (Absolute,X)     3/7

SLO      $1F       M <- (M >> 1) + A + C      (Absolute,X)     3/7

*   JSR      $20       Stack <- PC, PC <- Address (Absolute)       3/6

*   AND      $21       A <- (A) /\ M              (Ind,X)          2/6

JAM      $22       [locks up machine]         (Implied)        1/-

RLA      $23       M <- (M << 1) /\ (A)       (Ind,X)          2/8

*   BIT      $24       Z <- ~(A /\ M) N<-M7 V<-M6 (Z-Page)         2/3

*   AND      $25       A <- (A) /\ M              (Z-Page)         2/3

*   ROL      $26       C <- A7 & A <- A << 1 + C  (Z-Page)         2/5

RLA      $27       M <- (M << 1) /\ (A)       (Z-Page)         2/5'5

*   PLP      $28       A <- (Stack)               (Implied)        1/4

*   AND      $29       A <- (A) /\ M              (Immediate)      2/2

*   ROL      $2A       C <- A7 & A <- A << 1 + C  (Accumalator)    1/2

ANC      $2B       A <- A /\ M, C <- ~A7      (Immediate9      1/2

*   BIT      $2C       Z <- ~(A /\ M) N<-M7 V<-M6 (Absolute)       3/4

*   AND      $2D       A <- (A) /\ M              (Absolute)       3/4

*   ROL      $2E       C <- A7 & A <- A << 1 + C  (Absolute)       3/6

RLA      $2F       M <- (M << 1) /\ (A)       (Absolute)       3/6'5

*   BMI      $30       if N=1, PC = PC + offset   (Relative)       2/2'2

*   AND      $31       A <- (A) /\ M              ((Ind),Y)        2/5'1

JAM      $32       [locks up machine]         (Implied)        1/-

RLA      $33       M <- (M << 1) /\ (A)       ((Ind),Y)        2/8'5

NOP      $34       [no operation]             (Z-Page,X)       2/4

*   AND      $35       A <- (A) /\ M              (Z-Page,X)       2/4

*   ROL      $36       C <- A7 & A <- A << 1 + C  (Z-Page,X)       2/6

RLA      $37       M <- (M << 1) /\ (A)       (Z-Page,X)       2/6'5

*   SEC      $38       C <- 1                     (Implied)        1/2

*   AND      $39       A <- (A) /\ M              (Absolute,Y)     3/4'1

NOP      $3A       [no operation]             (Implied)        1/2

RLA      $3B       M <- (M << 1) /\ (A)       (Absolute,Y)     3/7'5

NOP      $3C       [no operation]             (Absolute,X)     3/4'1

*   AND      $3D       A <- (A) /\ M              (Absolute,X)     3/4'1

*   ROL      $3E       C <- A7 & A <- A << 1 + C  (Absolute,X)     3/7

RLA      $3F       M <- (M << 1) /\ (A)       (Absolute,X)     3/7'5

*   RTI      $40       P <- (Stack), PC <-(Stack) (Implied)        1/6

*   EOR      $41       A <- (A) \-/ M             (Ind,X)          2/6

JAM      $42       [locks up machine]         (Implied)        1/-

SRE      $43       M <- (M >> 1) \-/ A        (Ind,X)          2/8

NOP      $44       [no operation]             (Z-Page)         2/3

*   EOR      $45       A <- (A) \-/ M             (Z-Page)         2/3

*   LSR      $46       C <- A0, A <- (A) >> 1     (Absolute,X)     3/7

SRE      $47       M <- (M >> 1) \-/ A        (Z-Page)         2/5

*   PHA      $48       Stack <- (A)               (Implied)        1/3

*   EOR      $49       A <- (A) \-/ M             (Immediate)      2/2

*   LSR      $4A       C <- A0, A <- (A) >> 1     (Accumalator)    1/2

ASR      $4B       A <- [(A /\ M) >> 1]       (Immediate)      1/2

*   JMP      $4C       PC <- Address              (Absolute)       3/3

*   EOR      $4D       A <- (A) \-/ M             (Absolute)       3/4

*   LSR      $4E       C <- A0, A <- (A) >> 1     (Absolute)       3/6

SRE      $4F       M <- (M >> 1) \-/ A        (Absolute)       3/6

*   BVC      $50       if V=0, PC = PC + offset   (Relative)       2/2'2

*   EOR      $51       A <- (A) \-/ M             ((Ind),Y)        2/5'1

JAM      $52       [locks up machine]         (Implied)        1/-

SRE      $53       M <- (M >> 1) \-/ A        ((Ind),Y)        2/8

NOP      $54       [no operation]             (Z-Page,X)       2/4

*   EOR      $55       A <- (A) \-/ M             (Z-Page,X)       2/4

*   LSR      $56       C <- A0, A <- (A) >> 1     (Z-Page,X)       2/6

SRE      $57       M <- (M >> 1) \-/ A        (Z-Page,X)       2/6

*   CLI      $58       I <- 0                     (Implied)        1/2

*   EOR      $59       A <- (A) \-/ M             (Absolute,Y)     3/4'1

NOP      $5A       [no operation]             (Implied)        1/2

SRE      $5B       M <- (M >> 1) \-/ A        (Absolute,Y)     3/7

NOP      $5C       [no operation]             (Absolute,X)     3/4'1

*   EOR      $5D       A <- (A) \-/ M             (Absolute,X)     3/4'1

SRE      $5F       M <- (M >> 1) \-/ A        (Absolute,X)     3/7

*   RTS      $60       PC <- (Stack)              (Implied)        1/6

*   ADC      $61       A <- (A) + M + C           (Ind,X)          2/6

JAM      $62       [locks up machine]         (Implied)        1/-

RRA      $63       M <- (M >> 1) + (A) + C    (Ind,X)          2/8'5

NOP      $64       [no operation]             (Z-Page)         2/3

*   ADC      $65       A <- (A) + M + C           (Z-Page)         2/3

*   ROR      $66       C<-A0 & A<- (A7=C + A>>1)  (Z-Page)         2/5

RRA      $67       M <- (M >> 1) + (A) + C    (Z-Page)         2/5'5

*   PLA      $68       A <- (Stack)               (Implied)        1/4

*   ADC      $69       A <- (A) + M + C           (Immediate)      2/2

*   ROR      $6A       C<-A0 & A<- (A7=C + A>>1)  (Accumalator)    1/2

ARR      $6B       A <- [(A /\ M) >> 1]       (Immediate)      1/2'5

*   JMP      $6C       PC <- Address              (Indirect)       3/5

*   ADC      $6D       A <- (A) + M + C           (Absolute)       3/4

*   ROR      $6E       C<-A0 & A<- (A7=C + A>>1)  (Absolute)       3/6

RRA      $6F       M <- (M >> 1) + (A) + C    (Absolute)       3/6'5

*   BVS      $70       if V=1, PC = PC + offset   (Relative)       2/2'2

*   ADC      $71       A <- (A) + M + C           ((Ind),Y)        2/5'1

JAM      $72       [locks up machine]         (Implied)        1/-

RRA      $73       M <- (M >> 1) + (A) + C    ((Ind),Y)        2/8'5

NOP      $74       [no operation]             (Z-Page,X)       2/4

*   ADC      $75       A <- (A) + M + C           (Z-Page,X)       2/4

*   ROR      $76       C<-A0 & A<- (A7=C + A>>1)  (Z-Page,X)       2/6

RRA      $77       M <- (M >> 1) + (A) + C    (Z-Page,X)       2/6'5

*   SEI      $78       I <- 1                     (Implied)        1/2

*   ADC      $79       A <- (A) + M + C           (Absolute,Y)     3/4'1

NOP      $7A       [no operation]             (Implied)        1/2

RRA      $7B       M <- (M >> 1) + (A) + C    (Absolute,Y)     3/7'5

NOP      $7C       [no operation]             (Absolute,X)     3/4'1

*   ADC      $7D       A <- (A) + M + C           (Absolute,X)     3/4'1

*   ROR      $7E       C<-A0 & A<- (A7=C + A>>1)  (Absolute,X)     3/7

RRA      $7F       M <- (M >> 1) + (A) + C    (Absolute,X)     3/7'5

NOP      $80       [no operation]             (Immediate)      2/2

*   STA      $81       M <- (A)                   (Ind,X)          2/6

NOP      $82       [no operation]             (Immediate)      2/2

SAX      $83       M <- (A) /\ (X)            (Ind,X)          2/6

*   STY      $84       M <- (Y)                   (Z-Page)         2/3

*   STA      $85       M <- (A)                   (Z-Page)         2/3

*   STX      $86       M <- (X)                   (Z-Page)         2/3

SAX      $87       M <- (A) /\ (X)            (Z-Page)         2/3

*   DEY      $88       Y <- (Y) - 1               (Implied)        1/2

NOP      $89       [no operation]             (Immediate)      2/2

*   TXA      $8A       A <- (X)                   (Implied)        1/2

ANE      $8B       M <-[(A)\/$EE] /\ (X)/\(M) (Immediate)      2/2^4

*   STY      $8C       M <- (Y)                   (Absolute)       3/4

*   STA      $8D       M <- (A)                   (Absolute)       3/4

*   STX      $8E       M <- (X)                   (Absolute)       3/4

SAX      $8F       M <- (A) /\ (X)            (Absolute)       3/4

*   BCC      $90       if C=0, PC = PC + offset   (Relative)       2/2'2

*   STA      $91       M <- (A)                   ((Ind),Y)        2/6

JAM      $92       [locks up machine]         (Implied)        1/-

SHA      $93       M <- (A) /\ (X) /\ (PCH+1) (Absolute,X)     3/6'3

*   STY      $94       M <- (Y)                   (Z-Page,X)       2/4

*   STA      $95       M <- (A)                   (Z-Page,X)       2/4

SAX      $97       M <- (A) /\ (X)            (Z-Page,Y)       2/4

*   STX      $96       M <- (X)                   (Z-Page,Y)       2/4

*   TYA      $98       A <- (Y)                   (Implied)        1/2

*   STA      $99       M <- (A)                   (Absolute,Y)     3/5

*   TXS      $9A       S <- (X)                   (Implied)        1/2

SHS      $9B       X <- (A) /\ (X), S <- (X)  (Absolute,Y)     3/5

M <- (X) /\ (PCH+1)

SHY      $9C       M <- (Y) /\ (PCH+1)        (Absolute,Y)     3/5'3

*   STA      $9D       M <- (A)                   (Absolute,X)     3/5

SHX      $9E       M <- (X) /\ (PCH+1)        (Absolute,X)     3/5'3

SHA      $9F       M <- (A) /\ (X) /\ (PCH+1) (Absolute,Y)     3/5'3

*   LDY      $A0       Y <- M                     (Immediate)      2/2

*   LDA      $A1       A <- M                     (Ind,X)          2/6

*   LDX      $A2       X <- M                     (Immediate)      2/2

LAX      $A3       A <- M, X <- M             (Ind,X)          2/6

*   LDY      $A4       Y <- M                     (Z-Page)         2/3

*   LDA      $A5       A <- M                     (Z-Page)         2/3

*   LDX      $A6       X <- M                     (Z-Page)         2/3

LAX      $A7       A <- M, X <- M             (Z-Page)         2/3

*   TAY      $A8       Y <- (A)                   (Implied)        1/2

*   LDA      $A9       A <- M                     (Immediate)      2/2

*   TAX      $AA       X <- (A)                   (Implied)        1/2

LXA      $AB       X04 <- (X04) /\ M04        (Immediate)      1/2

A04 <- (A04) /\ M04

*   LDY      $AC       Y <- M                     (Absolute)       3/4

*   LDA      $AD       A <- M                     (Absolute)       3/4

*   LDX      $AE       X <- M                     (Absolute)       3/4

LAX      $AF       A <- M, X <- M             (Absolute)       3/4

*   BCS      $B0       if C=1, PC = PC + offset   (Relative)       2/2'2

*   LDA      $B1       A <- M                     ((Ind),Y)        2/5'1

JAM      $B2       [locks up machine]         (Implied)        1/-

LAX      $B3       A <- M, X <- M             ((Ind),Y)        2/5'1

*   LDY      $B4       Y <- M                     (Z-Page,X)       2/4

*   LDA      $B5       A <- M                     (Z-Page,X)       2/4

*   LDX      $B6       X <- M                     (Z-Page,Y)       2/4

LAX      $B7       A <- M, X <- M             (Z-Page,Y)       2/4

*   CLV      $B8       V <- 0                     (Implied)        1/2

*   LDA      $B9       A <- M                     (Absolute,Y)     3/4'1

*   TSX      $BA       X <- (S)                   (Implied)        1/2

LAE      $BB       X,S,A <- (S /\ M)          (Absolute,Y)     3/4'1

*   LDY      $BC       Y <- M                     (Absolute,X)     3/4'1

*   LDA      $BD       A <- M                     (Absolute,X)     3/4'1

*   LDX      $BE       X <- M                     (Absolute,Y)     3/4'1

LAX      $BF       A <- M, X <- M             (Absolute,Y)     3/4'1

*   CPY      $C0       (Y - M) -> NZC             (Immediate)      2/2

*   CMP      $C1       (A - M) -> NZC             (Ind,X)          2/6

NOP      $C2       [no operation]             (Immediate)      2/2

DCP      $C3       M <- (M)-1, (A-M) -> NZC   (Ind,X)          2/8

*   CPY      $C4       (Y - M) -> NZC             (Z-Page)         2/3

*   CMP      $C5       (A - M) -> NZC             (Z-Page)         2/3

*   DEC      $C6       M <- (M) - 1               (Z-Page)         2/5

DCP      $C7       M <- (M)-1, (A-M) -> NZC   (Z-Page)         2/5

*   INY      $C8       Y <- (Y) + 1               (Implied)        1/2

*   CMP      $C9       (A - M) -> NZC             (Immediate)      2/2

*   DEX      $CA       X <- (X) - 1               (Implied)        1/2

SBX      $CB       X <- (X)/\(A) - M          (Immediate)      2/2

*   CPY      $CC       (Y - M) -> NZC             (Absolute)       3/4

*   CMP      $CD       (A - M) -> NZC             (Absolute)       3/4

*   DEC      $CE       M <- (M) - 1               (Absolute)       3/6

DCP      $CF       M <- (M)-1, (A-M) -> NZC   (Absolute)       3/6

*   BNE      $D0       if Z=0, PC = PC + offset   (Relative)       2/2'2

*   CMP      $D1       (A - M) -> NZC             ((Ind),Y)        2/5'1

JAM      $D2       [locks up machine]         (Implied)        1/-

DCP      $D3       M <- (M)-1, (A-M) -> NZC   ((Ind),Y)        2/8

NOP      $D4       [no operation]             (Z-Page,X)       2/4

*   CMP      $D5       (A - M) -> NZC             (Z-Page,X)       2/4

*   DEC      $D6       M <- (M) - 1               (Z-Page,X)       2/6

DCP      $D7       M <- (M)-1, (A-M) -> NZC   (Z-Page,X)       2/6

*   CLD      $D8       D <- 0                     (Implied)        1/2

*   CMP      $D9       (A - M) -> NZC             (Absolute,Y)     3/4'1

NOP      $DA       [no operation]             (Implied)        1/2

DCP      $DB       M <- (M)-1, (A-M) -> NZC   (Absolute,Y)     3/7

NOP      $DC       [no operation]             (Absolute,X)     3/4'1

*   CMP      $DD       (A - M) -> NZC             (Absolute,X)     3/4'1

*   DEC      $DE       M <- (M) - 1               (Absolute,X)     3/7

DCP      $DF       M <- (M)-1, (A-M) -> NZC   (Absolute,X)     3/7

*   CPX      $E0       (X - M) -> NZC             (Immediate)      2/2

*   SBC      $E1       A <- (A) - M - ~C          (Ind,X)          2/6

NOP      $E2       [no operation]             (Immediate)      2/2

ISB      $E3       M <- (M) - 1,A <- (A)-M-~C (Ind,X)          3/8'1

*   CPX      $E4       (X - M) -> NZC             (Z-Page)         2/3

*   SBC      $E5       A <- (A) - M - ~C          (Z-Page)         2/3

*   INC      $E6       M <- (M) + 1               (Z-Page)         2/5

ISB      $E7       M <- (M) - 1,A <- (A)-M-~C (Z-Page)         2/5

*   INX      $E8       X <- (X) +1                (Implied)        1/2

*   SBC      $E9       A <- (A) - M - ~C          (Immediate)      2/2

*   NOP      $EA       [no operation]             (Implied)        1/2

SBC      $EB       A <- (A) - M - ~C          (Immediate)      1/2

*   SBC      $ED       A <- (A) - M - ~C          (Absolute)       3/4

*   CPX      $EC       (X - M) -> NZC             (Absolute)       3/4

*   INC      $EE       M <- (M) + 1               (Absolute)       3/6

ISB      $EF       M <- (M) - 1,A <- (A)-M-~C (Absolute)       3/6

*   BEQ      $F0       if Z=1, PC = PC + offset   (Relative)       2/2'2

*   SBC      $F1       A <- (A) - M - ~C          ((Ind),Y)        2/5'1

JAM      $F2       [locks up machine]         (Implied)        1/-

ISB      $F3       M <- (M) - 1,A <- (A)-M-~C ((Ind),Y)        2/8

NOP      $F4       [no operation]             (Z-Page,X)       2/4

*   SBC      $F5       A <- (A) - M - ~C          (Z-Page,X)       2/4

*   INC      $F6       M <- (M) + 1               (Z-Page,X)       2/6

ISB      $F7       M <- (M) - 1,A <- (A)-M-~C (Z-Page,X)       2/6

*   SED      $F8       D <- 1                     (Implied)        1/2

*   SBC      $F9       A <- (A) - M - ~C          (Absolute,Y)     3/4'1

NOP      $FA       [no operation]             (Implied)        1/2

ISB      $FB       M <- (M) - 1,A <- (A)-M-~C (Absolute,Y)     3/7

NOP      $FC       [no operation]             (Absolute,X)     3/4'1

*   SBC      $FD       A <- (A) - M - ~C          (Absolute,X)     3/4'1

*   INC      $FE       M <- (M) + 1               (Absolute,X)     3/7

ISB      $FF       M <- (M) - 1,A <- (A)-M-~C (Absolute,X)     3/7

'1 - Add one if address crosses a page boundry.

'2 - Add 1 if branch succeeds, or 2 if into another page.

'3 - If page boundry crossed then PCH+1 is just PCH

'4 - Sources disputed on exact operation, or sometimes does not work.

'5 - Full eight bit rotation (with carry)

Sources:

Programming the 6502, Rodney Zaks, (c) 1983 Sybex

Paul Ojala, Post to Comp.Sys.Cbm (po87553@cs.tut.fi / albert@cc.tut.fi)

D John Mckenna, Post to Comp.Sys.Cbm (gudjm@uniwa.uwa.oz.au)

Compiled by Craig Taylor (duck@pembvax1.pembroke.edu)

APPENDIX E

----------

; C64 Kernal Jump Table

;

ff81	jmp $ff5b	cint		Init Editor & Video Chips

ff84	jmp $fd23	ioinit		Init I/O Devices, Ports & Timers

ff87	jmp $fd50	ramtas		Init Ram & Buffers

ff8a	jmp $fd15	restor		Restore Vectors

ff8d	jmp $fd1a	vector		Change Vectors For User

ff90	jmp $fe18	setmsg		Control OS Messages

ff93	jmp $edb9	secnd		Send SA After Listen

ff96	jmp $edc7	tksa		Send SA After Talk

ff99	jmp $fe25	memtop		Set/Read System RAM Top

ff9c	jmp $fe34	membot		Set/Read System RAM Bottom

ff9f	jmp $ea87	scnkey		Scan Keyboard

ffa2	jmp $fe21	settmo		Set Timeout In IEEE

ffa5	jmp $ee13	acptr		Handshake Serial Byte In

ffa8	jmp $eddd	ciout		Handshake Serial Byte Out

ffab	jmp $edef	untalk		Command Serial Bus UNTALK

ffae	jmp $edfe	unlsn		Command Serial Bus UNLISTEN

ffb1	jmp $ed0c	listn		Command Serial Bus LISTEN

ffb4	jmp $ed09	talk		Command Serial Bus TALK

ffb7	jmp $fe07	readss		Read I/O Status Word

ffba	jmp $fe00	setlfs		Set Logical File Parameters

ffbd	jmp $fdf9	setnam		Set Filename

ffc0	jmp ($031a)	(iopen)		Open Vector [f34a]

ffc3	jmp ($031c)	(iclose)   	Close Vector [f291]

ffc6	jmp ($031e)	(ichkin)   	Set Input [f20e]

ffc9	jmp ($0320)	(ichkout)	Set Output [f250]

ffcc	jmp ($0322)	(iclrch)	Restore I/O Vector [f333]

ffcf	jmp ($0324)	(ichrin)	Input Vector, chrin [f157]

ffd2	jmp ($0326)	(ichrout)	Output Vector, chrout [f1ca]

ffd5	jmp $f49e	load		Load RAM From Device

ffd8	jmp $f5dd	save		Save RAM To Device

ffdb	jmp $f6e4	settim		Set Real-Time Clock

ffde	jmp $f6dd	rdtim		Read Real-Time Clock

ffe1	jmp ($0328)	(istop)		Test-Stop Vector [f6ed]

ffe4	jmp ($032a)	(igetin)	Get From Keyboad [f13e]

ffe7	jmp ($032c)	(iclall)	Close All Channels And Files [f32f]

ffea	jmp $f69b	udtim		Increment Real-Time Clock

ffed	jmp $e505	screen		Return Screen Organization

fff0	jmp $e50a	plot		Read / Set Cursor X/Y Position

fff3	jmp $e500	iobase		Return I/O Base Address

;fff6	Vectors

fff6	[5252]		-

fff8	[5942]		SYSTEM

;fffa	Transfer Vectors

fffa	[fe43]		NMI

fffc	[fce2]		RESET

fffe	[ff48]		IRQ

APPENDIX F

----------

BASIC KEYWORDS

COMMODORE BASIC KEYWORDS

Common Keywords (Tokens 80 - CB)

Tokens 80 to A2 represent action keywords, while codes B4 trough CA

are function keywords. AA - B3 are BASIC operators.

Token	Keyword

80	end

81	for

82	next

83	data

84	input#

85	input

86	dim

87	read

88	let

89	goto

8a	run

8b	if

8c	restore

8d	gosub

8e	return

8f	rem

90	stop

91	on

92	wait

93	load

94	save

95	verify

96	def

97	poke

98	print#

99	print

9a	cont

9b	list

9c	clr

9d	cmd

9e	sys

9f	open

a0	close

a1	get

a2	new

------------------

a3	tab(

a4	to

a5	fn

a6	spc(

a7	then

a8	not

a9	step

------------------

aa	+

ab	-

ac	*

ad	/

ae	^

af	and

b0	or

b1	>

b2	=

b3	<

------------------

b4	sgn

b5	int

b6	abs

b7	usr

b8	fre

b9	pos

ba	sqr

bb	rnd

bc	log

bd	exp

be	cos

bf	sin

c0	tan

c1	atn

c2	peek

c3	len

c4	str$

c5	val

c6	asc

c7	chr$

c8	left$

c9	right$

ca	mid$

------------------

cb	go

ff	pi

Extension Keywords (Tokens CC - FE)

The following codes are defined differently in each Basic version.

The leftmost column shows VIC Super Expander commands (CC trough DD).

Basic 3.5 and 7.0 differ in codes CE and FE, which are prefixes in 7.0,

whereas in 3.5 CE = rlum and FE is unused.

Codes CC to D4 (3.5, 7.0 and 10.0) are function keywords, and D5 trough

FA are action keywords.

Token	Keyword

2.0 Super	4.0		3.5/7.0		10.0

cc	key		concat		rgr		rgr  2)

cd	graphic		dopen		rclr		rclr 2)

ce	scnclr		dclose		rlum/*prefix*	*prefix*

cf	circle		record		joy		joy

d0	draw		header		rdot		rdot 2)

d1	region		collect		dec		dec

d2	color		backup		hex$		hex$

d3	point		copy		err$		err$

d4	sound		append		instr		instr

d5	char		dsave		else		else

d6	paint		dload		resume		resume

d7	rpot		catalog		trap		trap

d8	rpen		rename		tron		tron

d9	rsnd		scratch		troff		troff

da	rcolr		directory	sound		sound

db	rgr				vol		vol

dc	rjoy				auto		auto

dd	rdot				pudef		pudef

de					graphic		graphic

df					paint		paint 2)

e0					char		char

e1					box		box

e2					circle		circle

e3					gshape		paste 2)

e4					sshape		cut  2)

e5					draw		line

e6					locate		locate 2)

e7					color		color

e8					scnclr		scnclr

e9					scale		scale 2)

ea					help		help

eb					do		do

ec					loop		loop

ed					exit		exit

ee					directory	dir

ef					dsave		dsave

f0					dload		dload

f1					header		header

f2					scratch		scratch

f3					collect		collect

f4					copy		copy

f5					rename		rename

f6					backup		backup

f7					delete		delete

f8					renumber	renumber

f9					key		key

fa					monitor		monitor

--------------------------

fb					using		using

fc					until		until

fd					while		while

fe					*prefix* 	*prefix*

Prefixed Extension Keywords (Tokens CE02 - CE0A)

The following codes implement function keywords. Basics 7.0 and 10.0 only.

Token	Keyword

ce00

ce01

ce02	pot

ce03	bump

ce04	pen

ce05	rspos

ce06	rsprite

ce07	rspcolor

ce08	xor

ce09	rwindow

ce0a	pointer

Prefixed Extension Keywords (Tokens FE02 - FE26)

The following codes are for 7.0 and 10.0 only. Keywords in the

middle are commom.

Token		Keyword

7.0		   10.0

fe00

fe01

fe02		bank

fe03		filter

fe04		play

fe05		tempo

fe06		movspr

fe07		sprite

fe08		sprcolor

fe09		rreg

fe0a		envelope

fe0b		sleep

fe0c		catalog

fe0d		dopen

fe0e		append

fe0f		dclose

fe10		bsave

fe11		bload

fe12		record

fe13		concat

fe14		dverify

fe15		dclear

fe16		sprsav

fe17		collision

fe18		begin

fe19		bend

fe1a		window

fe1b		boot

fe1c		width  2)

fe1d		sprdef 2)

fe1e		quit 1) 2)

fe1f	 stash		 dma

fe20

fe21	 fetch		 dma

fe22

fe23	 swap		 dma

fe24		off 1) 2)

fe25		fast

fe26		slow

fe27			 type

fe28			 bverify

fe29			 ectory (diRectorY)

fe2a			 erase

fe2b			 find

fe2c			 change

fe2d			 set  3)

fe2e			 screen

fe2f			 polygon

fe30			 ellipse

fe31			 viewport 2)

fe32			 gcopy 2)

fe33			 pen

fe34			 palette

fe35			 dmode

fe36			 dpat

fe37			 pic  2)

fe38			 genlock

fe39			 foreground

fe3a

fe3b			 background

fe3c			 border

fe3d			 highlight

Notes:

1)	Gives "unimplemented command error" on BASIC 7.0

2)	Gives "unimplemented command error" on BASIC 10.0 v0.9

3)	Only 'set def' is implemented.

APPENDIX G

---------------

REU'S

The following is based on the Commodore 1764 user's manual (german

version)

Contents:

1) External RAM Access With REUs

2) RAM Expansion Controller (REC) Registers

3) How To Recognize The REU

4) Simple RAM Transfer

5) Additional Features

6) Transfer Speed

7) Interrupts

8) Executing Code In Expanded Memory

9) Other Useful Applications Of The REU

10) Comparision Of Bank Switching and DMA

1) _External RAM Access With REUs_

The REUs provide additional RAM for the C64/128. Three types of REUs

have been produced by Commodore. These are the 1700, 1764 and 1750 with

128, 256 and 512 KBytes built in RAM. However they can be extended up to

several MBytes.   The external memory can not be addressed directly by

the C64 with it's 16-bit address space. It has to be transferred from an

to the main memory of the C64. For that purpose there is a built in RAM

Expansion Controller (REC) which transfers memory between the C64 and the

REU using Direct Memory Access (DMA). It can also be used for other

purposes.

---

REU means Ram Expansion Unit. There are several different ones. The

official Commodore REU's are the 1700, 1764 and 1750 which are

respectively 128, 256 and 512Kb of memory (not directly addressable of

course). There seem to be hacks to expand these to 1Mb or even 2Mb. I

myself have recently made 512K in the 256K cartridge without any

difficulties. CLD, an american company makes clones of the 1750 and maybe

others. These clones are smaller than the originals but probably not as

expandable. I have a 1750 Clone (512Kb) and it seems to be 100%

compatible (no, not 99.9% but really 100%).

Furthermore there is the Georam expansion. This cartridge is ugly as hell

and only works with GEOS. I believe it's also 512K. In my opinion, the

real REU is better in every respect. (W. Lamee)

---

2) _RAM Expansion Controller (REC) Registers_

The REC is programmed by accessing it's registers, that appear memory

mapped in the I/O-area between $DF00 and $DF0A when a REU is connected

through the expansion port of the C64. They can be read and written to

like VIC- and SID-registers.

$DF00: STATUS REGISTER

various information can be obtained (read only)

Bit 7:     INTERRUPT PENDING  (1 = interrupt waiting to be served)

unnecessary

Bit 6:     END OF BLOCK  (1 = transfer complete)

unnecessary

Bit 5:     FAULT  (1 = block verify error)

Set if a difference between C64- and REU-memory areas was found

during a compare-command.

Bit 4:     SIZE  (1 = 256 KB)

Seems to indicate the size of the RAM-chips. It is set on 1764

and 1750 and clear on 1700.

Bits 3..0: VERSION

Contains 0 on my REU.

$DF01: COMMAND REGISTER

By writing to this register RAM transfer or comparision can be

executed.

Bit 7:     EXECUTE  (1 = transfer per current configuration)

This bit must be set to execute a command.

Bit 6:     reserved  (normally 0)

Bit 5:     LOAD  (1 = enable autoload option)

With autoload enabled the address and length registers (see

below) will be unchanged after a command execution.

Otherwise the address registers will be counted up to the

address off the last accessed byte of a DMA + 1,

and the length register will be changed (normally to 1).

Bit 4:     FF00

If this bit is set command execution starts immediately

after setting the command register.

Otherwise command execution is delayed until write access to

memory position $FF00

Bits 3..2: reserved  (normally 0)

Bits 1..0: TRANSFER TYPE

00 = transfer C64 -> REU

01 = transfer REU -> C64

10 = swap C64 <-> REU

11 = compare C64 - REU

$DF02..$DF03: C64 BASE ADDRESS

A 16-bit C64 - base address in low/high order.

$DF04..$DF06: REU BASE ADDRESS

This is a three byte address consisting of a low and

high byte and an expansion bank number.

Normally only bits 2..0 of the expansion bank are valid

(for a maximum of 512 KByte), the other bits are always

set. This must be different if more than 512 KByte are

installed.

$DF07..$DF08: TRANSFER LENGTH

This is a 16-bit value containing the number of bytes to

transfer or compare.

The value 0 stands for 64 Kbytes.

If the transfer length plus the C64 base address exceeds

64K the C64 address will overflow and cause C64 memory

from 0 on to be accessed.

If the transfer length plus the REU base address exceeds

512K the REU address will overflow and cause REU memory

from 0 on to be accessed.

$DF09: INTERRUPT MASK REGISTER

unnecessary

Bit 7:     INTERRUPT ENABLE  (1 = interrupt enabled)

Bit 6:     END OF BLOCK MASK  (1 = interrupt on end)

Bit 5:     VERIFY ERROR  (1 = interrupt on verify error)

Bits 4..0: unused (normally all set)

$DF0A: ADDRESS CONTROL REGISTER

Controlls the address counting during DMA.

If an address is fixed, not a memory block but always the same

byte addressed by the base address register is used for DMA.

Bit 7:     C64 ADDRESS CONTROL  (1 = fix C64 address)

Bit 6:     REU ADDRESS CONTROL  (1 = fix REU address)

Bits 5..0: unused (normally all set)

To access the REU-registers in assembly language it is convenient to

define labels something like this:

status   = $DF00

command  = $DF01

c64base  = $DF02

reubase  = $DF04

translen = $DF07

irqmask  = $DF09

control  = $DF0A

3) _How To Recognize The REU_

Normally the addresses between $DF00 and $DF0A are unused. So normally if

values are stored there they get lost. So if you write e.g. the values

1,2,3,... to $DF02..$DF08 and they don't stay there you can be sure that

no REU is connected. However if the values are there it could be because

another kind of module is connected that also uses these addresses.

Another problem is the recognition of the number of RAM banks (64 KByte

units) installed. The SIZE bit only tells that there are at least 2

(1700) or 4 (1764, 1750) banks installed. By trying to access & verify

bytes in as many RAM banks as possible the real size can be determined.

This can be seen in the source to "Dynamic memory allocation for the 128"

in Commodore Hacking Issue 2. (He) personally prefer(s) to let the user

choose if and which REU banks shall be used.

4) _Simple RAM Transfer_

Very little options of the REU are necessary for the main purposes of

RAM expanding.

Just set the base addresses, transfer length and then the command

register.

The following code transfers one KByte containing the screen

memory ($0400..$07FF) to address 0 in the REU:

lda #0

sta control ; to make sure both addresses are counted up

lda #<$0400

sta c64base

lda #>$0400

sta c64base + 1

lda #0

sta reubase

sta reubase + 1

sta reubase + 2

lda #<$0400

sta translen

lda #>$0400

sta translen + 1

lda #%10010000;  c64 -> REU with immediate execution

sta command

To transfer the memory back to the C64 replace "lda #%10010000"

by "lda #%10010001".

I think that this subset of 17xx functions would be enough for a

reasonable RAM expansion. However if full compatibility with 17xx REUs

is desired also the more complicated functions have to be implemented.

5) _Additional Features_

Swapping Memory

With the swap-command memory between 17xx and C64 is exchanged. The

programming is the same as in simple RAM transfer.

Comparing Memory

No RAM is transferred but the number of bytes specified in the

transfer length register is compared. If there are differences the

FAULT-bit of the status register is set. This bit is cleared by reading

the status register which has to be done before comparing to get valid

information.

Using All C64 Memory

C64 memory is accessed by the REU normally in the memory configuration

existing during writing to the command register. However in order to be

able to write to the command register the I/O-area has to be active.

If RAM between $D000 and $DFFF or character ROM shall be used it is

possible to delay the execution of the command by storing a command byte

with bit 4 ("FF00") cleared. The command will then be executed

by writing any value to address $FF00.

Example:

< Set base addresses and transfer length >

lda #%10000000 ; transfer C64 RAM -> REU delayed

sta command

sei

lda $01

and #$30

sta $01 ; switch on 64 KByte RAM

lda $FF00 ; to not change the contents of $FF00

sta $FF00 ; execute DMA

lda $01

ora #$37

sta $01 ; switch on normal configuration

cli

6) _Transfer Speed_

During DMA the CPU is halted and the memory access cycles normally

available for the CPU are now used to access one byte each. So with

screen and sprites switched off in every clock cycle (985248 per second

on PAL machines) a byte is transferred. If screen is on or sprites are

enabled transfer is a bit slower, as the VIC exclusively accesses RAM

sometimes. An exact description of those "missing cycles" can be found

in Commodore Hacking Issue 3.

Comparing memory areas is as fast as transfers. (Comparison is stopped

once the first difference is found.)

Swapping memory is only half as fast, as for every bytes two C64 memory

accesses (read & write) are necessary.

7) _Interrupts_

By setting certain bits in the interrupt mask register IRQs at the end

of a DMA can be selected. However as the CPU is halted during DMA it

will always be finished after the store instruction into the command

register or $FF00. So there is no need to check for an "END OF BLOCK"

(bit 6 of status register) or to enable an interrupt.

8) _Executing Code In Expanded Memory_

Code in external memory has always to be copied into C64 memory to be

executed. This is a disadvantage against bank switching systems. However

bank switching can be simulated by the SWAP command. This is done e.g.

in RAMDOS where only 256 bytes of C64 memory are occupied, the 6 KByte

RAM disk driver is swapped in whenever needed. Probably too much

swapping is the reason for RAMDOS to be not really fast at sequential

file access.

9) _Other Useful Applications Of The REU_

The REC is not only useful for RAM transfer and comparison.

One other application (used in GEOS) is to copy C64 RAM areas

by first transferring it to the REU and then transferring it back into

the desired position in C64 memory. Due to the fast DMA this is about 5

times faster than copying memory with machine language instructions.

Interesting things can be done by fixing base addresses. Large C64

areas can be filled very fast with a single byte value by fixing the REU

base address. Thus it is also possible to find the end of an area

containing equal bytes very fast e.g. for data compression.

Fixing the C64 base address is interesting if an I/O-port is used, as

data can be written out faster than normally possible.

It would be possible to use real bitmap graphics in the upper and lower

screen border by changing the "magic byte" (highest by the VIC addressed

byte) in every clock cycle during the border switched off.

Generally the REC could be used as graphics accelerator e.g. to

copy bitmap areas or to copy data fast into the VIC-addressable

16 KByte area.

10) _Comparision Of Bank Switching and DMA_

When comparing bank switching and DMA for memory expansion I think DMA

is the more comfortable methode to program and also is faster in most

cases. The disadvantage with code execution not possible in external

memory could be minimized by copying only the necessary parts into C64

memory. Executing the code will take much more time than copying it

into C64 memory.

APPENDIX H

-----------

ABOUT THE PROCESSOR CHIP

C=  Commodore Semiconductor Group

Microprocessors

Description

The 6500/8500 Series family includes a range of software compatible micropro-

cessors which provide a selection of addressable memory range, interrupt input

options and on-chip oscillators and drivers. All of the microprocessors within

the group are directly bus compatible with the MC6800 series IC's.

The family includes ten microprocessors with on-board clock oscillators and

seven microprocessors driven by external clocks. The on-chip clock versions

are aimed at high performance, low cost applications where single phase crystal

or RC inputs provide the time base. The external clock versions are geared for

multiprocessor system applications where maximum timing control is mandatory.

Features

Single +5 volt supply

N channel, silicon gate, depletion load technology

Tri-state address bus, data bus and R/W controlled by AEC input

Direct memory access capability

"Ready" input (for single cycle execution)

56 Instructions with 13 addressing modes

8 bit parallel processing

Decimal and binary arithmetic

True indexing capability

8 bit Bi-directional Data Bus

Programmable Stack Pointer

Available Microprocessors

Device	*Clocks	Pins	IRQ NMI RDY	Port	Address	AEC Sync Speed (MHz)

6502	  O	40	 X   X   X	-	64K	 -   X	  1,2,3,4

65CE02	  O	40	 X   X   X	-	64K	 -   X	  0 - 10

6503	  O	28	 X   X   -	-	 4K	 -   -	  1,2,3,4

6504	  O	28	 X   -   -	-	 8K	 -   -	  1,2,3,4

6505	  O	28	 X   -   X	-	 4K	 -   -	  1,2,3,4

6506	  O	28	 X   -   -	-	 4K	 -   -	  1,2,3,4

6507	  O	28	 -   -   X	-	 8K	 -   -	  1,2,3,4

6508	  E	40	 X   -   -	8	64K	 X   -	  1,2,3

6509	  E	40	 X   X   X	**	1 M	 X   X	  1,2,3

6510	 O,E	40	 X   X   X	6,8	64K	 X   -	  1,2,3,4

6512	  E	40	 X   X   X	-	64K	 -   X	  1,2,3,4

6513	  E	28	 X   X   -	-	 4K	 -   -	  1,2,3,4

6514	  E	28	 X   -   -	-	 8K	 -   -	  1,2,3,4

6515	  E	28	 X   -   X	-	 4K	 -   -	  1,2,3,4

8501	  O	40	 X   -   X	7	64K	 X   -	  1,2,3

8502	  O	40	 X   X   X	7	64K	 X   -	  1,2,3,4

8503	  O	40	 X   -   -	8	64K	 X   -	  1,2,3,4

* O - On chip clocks, E - External Clocks

** Four extended address pins expand memory capacity to one megabyte.

Pinout

Pin	6502		6510/8500	8502

1	Vss		Phi0 in		Phi0 in

2	RDY		RDY		RDY

3	Phi1 out	/IRQ		/IRQ

4	/IRQ		/NMI		/NMI

5	NC		AEC		AEC

6	/NMI		Vcc		Vcc

7	Sync		A0		A0

8	Vcc		A1		A1

9	AB0		A2		A2

10	AB1		A3		A3

11	AB2		A4		A4

12	AB3		A5		A5

13	AB4		A6		A6

14	AB5		A7		A7

15	AB6		A8		A8

16	AB7		A9		A9

17	AB8		A10		A10

18	AB9		A11		A11

19	AB10		A12		A12

20	AB11		A13		A13

21	Vss		GND		GND

22	AB12		A14		A14

23	AB13		A15		A15

24	AB14		P5		P6

25	AB15		P4		P5

26	D7		P3		P4

27	D6		P2		P3

28	D5		P1		P2

29	D4		P0		P1

30	D3		D7		P0

31	D2		D6		D7

32	D1		D5		D6

33	D0		D4		D5

34	R/W		D3		D4

35	NC		D2		D3

36	NC		D1		D2

37	Phi0 in		D0		D1

38	SO		R/W		D0

39	Phi2 out	Phi2 out	R/W

40	/RES		/RES		/RES

APPENDIX I

--------------

DIFFERENCES IN PROCESSORS

-----------------------------

I told you that I'd come back with something like this, so here it is!

This is taken from CHacking..

"Q $03F) Now, for those into 6502 machine language.  What instruction was not

available on the first 6502 chips?

A $03F) ROR (ROtate Right) was not available until after June, 1976.  However,

all Commodore VICs and C64s should have this instruction.  Some people

gave instructions that are found on the 65c02, designed by Western

Design Center, and licensed to many companies.  However, the 65c02

itself occurs in two flavors, and neither are used in any stock

Commodore product I know of."

Here's another interesting tidbit (from CHACKING)

It seems that the "6510 internal registers were grafted onto a 6502 core

processor."

64 KERNAL ROM DIFFERENCES

Date: Fri Jun 17 16:38:46 1994

Received: from funet.fi by oulu.fi (4.1/SMI-4.1)

6.2 Commodore 64 KERNAL ROM versions.

Below is information on differences between the Commodore 64

KERNAL revisions R1, R2, R3 and the Commodore SX-64 and the

Commodore 4064 ROMs.  The chronological order must be R1, R2, 4064,

R3 and SX-64.

The KERNAL ROM R1 was obviously used only in early NTSC systems.

It lacks the PAL/NTSC detection, and always uses white color while

clearing the screen.  The white color feature is from the VIC-20

ROM, but the VIC had a white background by default.  Thus, this

feature can be listed as a bug.  The CIA 1 timer A will always

divide the system clock through $411C == 16668.  The other ROMs use

the values $4026 an $4296, depending on the system version

(PAL/NTSC), so their interrupt frequency is 985248 Hz / 16422 ==

59.996 Hz or 1022727 Hz / 17046 == 59.998 Hz.  Note that both

clock divisor values differ from the value used in the KERNAL R1.

The PAL/NTSC flag ($2A6) affects the RS-232 timer settings as well.

It seems that the new RS-232 tables for the PAL have been created on

the upper BASIC interpreter area ($E000--$E4FF), from the address

$E4EC on. Surprisingly also the original NTSC tables have been

changed.  Very probably the units running the KERNAL R1 had a slower

clock frequency.  Extrapolating from the interrupt timer values,

they ran at 1.0000 MHz.  Now this makes sense, since the first

(NTSC) video chips had 262 lines per frame and 64 cycles per line.

The frame rate was thus 1 MHz / 262 / 64 == 59.637 Hz.  The newer

NTSC units run at 1022727 Hz and draw 263 lines per frame and use 65

cycles per line.  This produces a frame rate of 59.826 Hz.  Well,

now it is very obvious that there has been at least one mother board

type that has only been used on NTSC units.  Probably the processor

clock was created from a 8 MHz chrystal frequency, which served as

the dot clock.  The latter NTSC units generate the processor clock

by dividing the chrystal frequency of 14318181 Hz by 14, and the dot

clock will be generated by octacoupling the processor clock.

The PAL systems have been developed later, and they always run at

the same clock frequency, 17734472 Hz / 18.  The frame rate has

always been 17734472 Hz / 312 / 63 == 50.125 Hz on those puppies.

The changes in the latter ROM revisions were mainly cosmetical.

There were some bugs corrected in the R3 revision, though.

Format for list:

Address: 901227-01 (Commodore 64 KERNAL R1, $FF80 content $AA)

901227-02 (Commodore 64 KERNAL R2, $FF80 content $00)

901227-03 (Commodore 64 KERNAL R3, $FF80 content $03)

??????-?? (SX-64 or DX-64 KERNAL, $FF80 content $43)

??????-?? (4064 aka PET 64 aka Educator 64, $FF80 content $64)

E119:  C9, FF

AD, E4

AD, E4

AD, E4

AD, E4

E42D:  20, 1E, AB

20, 1E, AB

20, 1E, AB

20, 1E, AB

4C, 41, E4

E477:  20, 20, 2A, 2A, 2A, 2A, 20, 43, 4F, 4D, 4D, 4F, 44, 4F, 52, 45,

20, 20, 2A, 2A, 2A, 2A, 20, 43, 4F, 4D, 4D, 4F, 44, 4F, 52, 45,

20, 20, 2A, 2A, 2A, 2A, 20, 43, 4F, 4D, 4D, 4F, 44, 4F, 52, 45,

20, 20, 20, 2A, 2A, 2A, 2A, 2A, 20, 20, 53, 58, 2D, 36, 34, 20,

2A, 2A, 2A, 2A, 20, 43, 4F, 4D, 4D, 4F, 44, 4F, 52, 45, 20, 34,

-:     20, 36, 34, 20, 42, 41, 53, 49, 43, 20, 56, 32, 20, 2A, 2A, 2A,

20, 36, 34, 20, 42, 41, 53, 49, 43, 20, 56, 32, 20, 2A, 2A, 2A,

20, 36, 34, 20, 42, 41, 53, 49, 43, 20, 56, 32, 20, 2A, 2A, 2A,

42, 41, 53, 49, 43, 20, 56, 32, 2E, 30, 20, 20, 2A, 2A, 2A, 2A,

30, 36, 34, 20, 20, 42, 41, 53, 49, 43, 20, 56, 32, 2E, 30, 20,

-:     2A, 0D, 0D, 20, 36, 34, 4B, 20, 52, 41, 4D, 20, 53, 59, 53, 54,

2A, 0D, 0D, 20, 36, 34, 4B, 20, 52, 41, 4D, 20, 53, 59, 53, 54,

2A, 0D, 0D, 20, 36, 34, 4B, 20, 52, 41, 4D, 20, 53, 59, 53, 54,

2A, 0D, 0D, 20, 36, 34, 4B, 20, 52, 41, 4D, 20, 53, 59, 53, 54,

2A, 2A, 2A, 2A, 0D, 0D, 00, 20, 20, 20, 20, 20, 20, 20, 20, 20,

-:     45, 4D, 20, 20, 00, 2B

45, 4D, 20, 20, 00, 5C

45, 4D, 20, 20, 00, 81

45, 4D, 20, 20, 00, B3

20, 20, 20, 20, 20, 63

E4AD:  AA, AA, AA, AA, AA, AA, AA, AA, AA, AA

48, 20, C9, FF, AA, 68, 90, 01, 8A, 60

48, 20, C9, FF, AA, 68, 90, 01, 8A, 60

48, 20, C9, FF, AA, 68, 90, 01, 8A, 60

48, 20, C9, FF, AA, 68, 90, 01, 8A, 60

E4C8:  AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA,

AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA,

AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, 85, A9, A9, 01, 85,

AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, 85, A9, A9, 01, 85,

2C, 86, 02, 30, 0A, A9, 00, A2, 0E, 9D, 20, D0, CA, 10, FA, 4C,

-:     AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA,

AA, AA, AD, 21, D0, 91, F3, 60, 69, 02, A4, 91, C8, D0, 04, C5,

AB, 60, AD, 86, 02, 91, F3, 60, 69, 02, A4, 91, C8, D0, 04, C5,

AB, 60, AD, 86, 02, 91, F3, 60, 69, 02, A4, 91, C8, D0, 04, C5,

87, EA, AD, 21, D0, 91, F3, 60, 69, 02, A4, 91, C8, D0, 04, C5,

-:     AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA,

A1, D0, F7, 60, 19, 26, 44, 19, 1A, 11, E8, 0D, 70, 0C, 06, 06,

A1, D0, F7, 60, 19, 26, 44, 19, 1A, 11, E8, 0D, 70, 0C, 06, 06,

A1, D0, F7, 60, 19, 26, 44, 19, 1A, 11, E8, 0D, 70, 0C, 06, 06,

A1, D0, F7, 60, 19, 26, 44, 19, 1A, 11, E8, 0D, 70, 0C, 06, 06,

-:     AA, AA, AA, AA, AA, AA, AA, AA

D1, 02, 37, 01, AE, 00, 69, 00

D1, 02, 37, 01, AE, 00, 69, 00

D1, 02, 37, 01, AE, 00, 69, 00

D1, 02, 37, 01, AE, 00, 69, 00

E535:  0E

0E

0E

06

01

E57C:  B5, D9, 29, 03, 0D, 88, 02, 85, D2, BD, F0, EC, 85, D1, A9, 27,

B5, D9, 29, 03, 0D, 88, 02, 85, D2, BD, F0, EC, 85, D1, A9, 27,

20, F0, E9, A9, 27, E8, B4, D9, 30, 06, 18, 69, 28, E8, 10, F6,

20, F0, E9, A9, 27, E8, B4, D9, 30, 06, 18, 69, 28, E8, 10, F6,

20, F0, E9, A9, 27, E8, B4, D9, 30, 06, 18, 69, 28, E8, 10, F6,

-:     E8, B4, D9, 30, 06, 18, 69, 28, E8, 10, F6, 85, D5, 60

E8, B4, D9, 30, 06, 18, 69, 28, E8, 10, F6, 85, D5, 60

85, D5, 4C, 24, EA, E4, C9, F0, 03, 4C, ED, E6, 60, EA

85, D5, 4C, 24, EA, E4, C9, F0, 03, 4C, ED, E6, 60, EA

85, D5, 4C, 24, EA, E4, C9, F0, 03, 4C, ED, E6, 60, EA

E5EF:  09

09

09

0F

09

E5F4:  E6, EC

E6, EC

E6, EC

D7, F0

E6, EC

E622:  ED, E6

ED, E6

91, E5

91, E5

91, E5

EA07:  A9, 20, 91, D1, A9, 01, 91, F3, 88, 10, F5, 60

A9, 20, 91, D1, 20, DA, E4, EA, 88, 10, F5, 60

20, DA, E4, A9, 20, 91, D1, 88, 10, F6, 60, EA

20, DA, E4, A9, 20, 91, D1, 88, 10, F6, 60, EA

A9, 20, 91, D1, 20, DA, E4, EA, 88, 10, F5, 60

ECCA:  1B, 00

9B, 37

9B, 37

9B, 37

9B, 37

ECD2:  00

0F

0F

0F

0F

ECD9:  0E, 06, 01, 02, 03, 04, 00, 01, 02, 03, 04, 05, 06, 07

0E, 06, 01, 02, 03, 04, 00, 01, 02, 03, 04, 05, 06, 07

0E, 06, 01, 02, 03, 04, 00, 01, 02, 03, 04, 05, 06, 07

03, 01, 01, 02, 03, 04, 00, 01, 02, 03, 04, 05, 06, 07

00, 00, 00, 00, 00, 00, 00, 00, 00, 00, 00, 00, 00, 00

EF94:  85, A9, 60

85, A9, 60

4C, D3, E4

4C, D3, E4

85, A9, 60

F0D8:  0D, 50, 52, 45, 53, 53, 20, 50, 4C, 41, 59, 20, 4F, 4E, 20

0D, 50, 52, 45, 53, 53, 20, 50, 4C ,41, 59, 20, 4F, 4E, 20

0D, 50, 52, 45, 53, 53, 20, 50, 4C ,41, 59, 20, 4F, 4E, 20

4C, 4F, 41, 44, 22, 3A, 2A, 22, 2C, 38, 0D, 52, 55, 4E, 0D

0D, 50, 52, 45, 53, 53, 20, 50, 4C ,41, 59, 20, 4F, 4E, 20

F387:  03

03

03

08

03

F428:  D0, 0B, AD, 95, 02, 0A, A8, AD, 96, 02, 4C, 3F, F4, 0A, AA, BD,

F0, 1C, 0A, AA, AD, A6, 02, D0, 09, BC, C1, FE, BD, C0, FE, 4C,

F0, 1C, 0A, AA, AD, A6, 02, D0, 09, BC, C1, FE, BD, C0, FE, 4C,

F0, 1C, 0A, AA, AD, A6, 02, D0, 09, BC, C1, FE, BD, C0, FE, 4C,

F0, 1C, 0A, AA, AD, A6, 02, D0, 09, BC, C1, FE, BD, C0, FE, 4C,

-:     C0, FE, 0A, A8, BD, C1, FE, 2A, 48, 98, 69, C8, 8D, 99, 02, 68,

40, F4, BC, EB, E4, BD, EA, E4, 8C, 96, 02, 8D, 95, 02, AD, 95,

40, F4, BC, EB, E4, BD, EA, E4, 8C, 96, 02, 8D, 95, 02, AD, 95,

40, F4, BC, EB, E4, BD, EA, E4, 8C, 96, 02, 8D, 95, 02, AD, 95,

40, F4, BC, EB, E4, BD, EA, E4, 8C, 96, 02, 8D, 95, 02, AD, 95,

-:     69, 00, 8D, 9A, 02

02, 0A, 20, 2E, FF

02, 0A, 20, 2E, FF

02, 0A, 20, 2E, FF

02, 0A, 20, 2E, FF

F459:  4C

20

20

20

20

F4B7:  7B

7B

7B

F7

7B

F5F9:  5F

5F

5F

F7

5F

F762:  91, C9, FF, F0, FA

A1, 20, E0, E4, EA

A1, 20, E0, E4, EA

A1, 20, E0, E4, EA

A1, 20, E0, E4, EA

F81F:  2F

2F

2F

2F

2B

F82C:  2F

2F

2F

2F

2B

FCFC:  18, E5

5B, FF

5B, FF

5B, FF

5B, FF

FDDD:  A9, 1B, 8D, 04, DC, A9, 41, 8D, 05, DC, A9, 81, 8D, 0D, DC, AD,

AD, A6, 02, F0, 0A, A9, 25, 8D, 04, DC, A9, 40, 4C, F3, FD, A9,

AD, A6, 02, F0, 0A, A9, 25, 8D, 04, DC, A9, 40, 4C, F3, FD, A9,

AD, A6, 02, F0, 0A, A9, 25, 8D, 04, DC, A9, 40, 4C, F3, FD, A9,

AD, A6, 02, F0, 0A, A9, 25, 8D, 04, DC, A9, 40, 4C, F3, FD, A9,

-:     0E, DC, 29, 80, 09, 11, 8D, 0E, DC, 4C, 8E, EE

95, 8D, 04, DC, A9, 42, 8D, 05, DC, 4C, 6E, FF

95, 8D, 04, DC, A9, 42, 8D, 05, DC, 4C, 6E, FF

95, 8D, 04, DC, A9, 42, 8D, 05, DC, 4C, 6E, FF

95, 8D, 04, DC, A9, 42, 8D, 05, DC, 4C, 6E, FF

FEC2:  AC, 26, A7, 19, 5D, 11, 1F, 0E, A1, 0C, 1F, 06, DD, 02, 3D, 01,

C1, 27, 3E, 1A, C5, 11, 74, 0E, ED, 0C, 45, 06, F0, 02, 46, 01,

C1, 27, 3E, 1A, C5, 11, 74, 0E, ED, 0C, 45, 06, F0, 02, 46, 01,

C1, 27, 3E, 1A, C5, 11, 74, 0E, ED, 0C, 45, 06, F0, 02, 46, 01,

C1, 27, 3E, 1A, C5, 11, 74, 0E, ED, 0C, 45, 06, F0, 02, 46, 01,

-:     B2, 00, 6C

B8, 00, 71

B8, 00, 71

B8, 00, 71

B8, 00, 71

FF08:  93, 02, 29, 0F, D0, 0C, AD, 95, 02, 8D, 06, DD, AD, 96, 02, 4C,

95, 02, 8D, 06, DD, AD, 96, 02, 8D, 07, DD, A9, 11, 8D, 0F, DD,

95, 02, 8D, 06, DD, AD, 96, 02, 8D, 07, DD, A9, 11, 8D, 0F, DD,

95, 02, 8D, 06, DD, AD, 96, 02, 8D, 07, DD, A9, 11, 8D, 0F, DD,

95, 02, 8D, 06, DD, AD, 96, 02, 8D, 07, DD, A9, 11, 8D, 0F, DD,

-:     25, FF, 0A, AA, BD, C0, FE, 8D, 06, DD, BD, C1, FE, 8D, 07, DD,

A9, 12, 4D, A1, 02, 8D, A1, 02, A9, FF, 8D, 06, DD, 8D, 07, DD,

A9, 12, 4D, A1, 02, 8D, A1, 02, A9, FF, 8D, 06, DD, 8D, 07, DD,

A9, 12, 4D, A1, 02, 8D, A1, 02, A9, FF, 8D, 06, DD, 8D, 07, DD,

A9, 12, 4D, A1, 02, 8D, A1, 02, A9, FF, 8D, 06, DD, 8D, 07, DD,

-:     A9, 11, 8D, 0F, DD, A9, 12, 4D, A1, 02, 8D, A1, 02, A9, FF, 8D,

AE, 98, 02, 86, A8, 60, AA, AD, 96, 02, 2A, A8, 8A, 69, C8, 8D,

AE, 98, 02, 86, A8, 60, AA, AD, 96, 02, 2A, A8, 8A, 69, C8, 8D,

AE, 98, 02, 86, A8, 60, AA, AD, 96, 02, 2A, A8, 8A, 69, C8, 8D,

AE, 98, 02, 86, A8, 60, AA, AD, 96, 02, 2A, A8, 8A, 69, C8, 8D,

-:     06, DD, 8D, 07, DD, AE, 98, 02, 86, A8, 60

99, 02, 98, 69, 00, 8D, 9A, 02, 60, EA, EA

99, 02, 98, 69, 00, 8D, 9A, 02, 60, EA, EA

99, 02, 98, 69, 00, 8D, 9A, 02, 60, EA, EA

99, 02, 98, 69, 00, 8D, 9A, 02, 60, EA, EA

FF5B:  AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA,

20, 18, E5, AD, 12, D0, D0, FB, AD, 19, D0, 29, 01, 8D, A6, 02,

20, 18, E5, AD, 12, D0, D0, FB, AD, 19, D0, 29, 01, 8D, A6, 02,

20, 18, E5, AD, 12, D0, D0, FB, AD, 19, D0, 29, 01, 8D, A6, 02,

20, 18, E5, AD, 12, D0, D0, FB, AD, 19, D0, 29, 01, 8D, A6, 02,

-:     AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA, AA,

4C, DD, FD, A9, 81, 8D, 0D, DC, AD, 0E, DC, 29, 80, 09, 11, 8D,

4C, DD, FD, A9, 81, 8D, 0D, DC, AD, 0E, DC, 29, 80, 09, 11, 8D,

4C, DD, FD, A9, 81, 8D, 0D, DC, AD, 0E, DC, 29, 80, 09, 11, 8D,

4C, DD, FD, A9, 81, 8D, 0D, DC, AD, 0E, DC, 29, 80, 09, 11, 8D,

-:     AA, AA, AA, AA, AA

0E, DC, 4C, 8E, EE

0E, DC, 4C, 8E, EE

0E, DC, 4C, 8E, EE

0E, DC, 4C, 8E, EE

FF80:  AA

00

03

43

64

FF82:  18, E5

53, FF

53, FF

53, FF

53, FF

FFF8:  42, 59

42, 59

42, 59

42, 59

00, 00

APPENDIX J

----------

CHIP INFORMATION CHART

IC'S

-----

LOCATION	IC NUMBER		DESCRIPTION

--------	---------		-----------

U1		6526 CIA #1		COMPLEX INTERFACE ADAPTER

U2		6526 CIA #2		"

U3		901226-01		NMOS 8192X8 STATIC BASIC ROM

U4		901227-XX		NMOS 8192X8 STATIC KERNAL ROM

U5		901225-01		NMOS 4096X8 STATIC CHARACTER ROM

U6 		2114-30L/MCM2114P20	NMOS 1024X8 STATIC RAM

U7		6510			NMOS MPU (CPU)

U8		7406N/M53206P		QUAD OPERATIONAL AMPLIFIER

U9		4164-2/MK4564N-20	NMOS 65536X1-BIT DYNAMIC RAM

U10		4164-2/MK4564N-20	NMOS 65536X1-BIT DYNAMIC RAM

U11		4164-2/MK4564N-20	NMOS 65536X1-BIT DYNAMIC RAM

U12		4164-2/MK4564N-20	NMOS 65536X1-BIT DYNAMIC RAM

U13		74LS257			QUAD 2-INPUT TRI-STATE MULTIPLEXER

U14		74LS258			TTL DIGITAL MULTIPLEXER

U15		74LS139			DUAL 2/4 DECODER DEMULTIPLEXER

U16		4066			CMOS QUAD ANALOG SWITCH

U17		82S100			FIELD PROGRAMMABLE PLA

U18		6581 SID		SOUND INTERFACE DEVICE

U19		6567 VIC		VIDEO INTERFACE CHIP

U20		556/MC3456		DUAL 555 TIMER

U21 		4164-2 RAM		NMOS 65536X1-BIT DYNAMIC RAM

U22		4164-2 RAM		NMOS 65536X1-BIT DYNAMIC RAM

U23		4164-2 RAM		NMOS 65536X1-BIT DYNAMIC RAM

U24		4164-2 RAM		NMOS 65536X1-BIT DYNAMIC RAM

U25		74LS257			QUAD 2-INPUT TRI-STATE MULTIPLEXER

U26		74LS373			8-BIT TRANSPARENT LATCH

U27		75LS08			QUAD 2-INPUT AND

U28		4066			CMOS ANALOG SWITCH

U29		74LS74			QUAD D FLIP-FLOP

U30		74LS193			BINARY UP/DOWN COUNTER

U31		74LS629N		DUAL VOLTAGE CONTROLLER OSCILLATOR

U32		MC4044			TTL PHASE FREQUENCY DETECTOR

OTHER COMPONENTS:

LOCATION	DEVICE			DESCRIPTION

--------	------			-----------

CR1		1N4371			2.7-VOLT ZENER DIODE

CR2		1N755			7.5-VOLT ZENER DIODE

CR3		1N914			SIGNAL DIODE

CR4		VM08 (P/S)		BRIDGE RECTIFIER DIODE

CR5		1N4001 (P/S)		POWER DIODE

CR6		1N4001 (P/S)		POWER DIODE

Q1		2N4401			TRANSISTOR

Q2		2N3904			"

Q3		TP29B			"

Q4		PN2222			"

Q5		PN2222			"

Q6		PN2222			"

Q7		PN2222A			"

Q8		PN2222			"

VR1		MD7812CT/UA7812UC	FIXED POSITIVE LINEAR VOLTAGE REG.

VR2		MC7805CT		" WITH 1500 mA OUTPUT

APPENDIX K

----------

SPECIFICATIONS OF THE COMMODORE 64

MANUFACTURER:		COMMODORE BUSINESS SYSTEMS

1200 WILSON DRIVE

WEST CHESTER, PA 19380

SIZE:			2.75"X15.9"X8.0"

WEIGHT:			4.1 LBS.

POWER REQUIRED:		LESS THAN 20 WATTS 8.5	WATTS AT 5.V DC

MPU: 			COMMODORE 6510 MPU

DATA WORD SIZE:		8-BITS

CPU CLOCK SPEED:	1.023 MHz

MEMORY SIZE:		64K

MASS STORAGE CAPABILITY:

UP TO 4 VIC-1541 DISK DRIVES

DATA CASSETTE RECORDER

KEYBOARD SIZE: 		65 KEYS

157 CHARACTER CODES

TEXT DISPLAY:		40 UPPERCASE CHARACTERS (2-CHAR SETS)

24 LINES

GRAPHICS CAPABILITY:	LOW RES - 160 X 200 PIXELS

HIGH RES - 320 X 200 PIXELS

USER DEFINED SPRITE GRAPHICS

COLOR CAPABILITY:	16 COLORS

INPUT/OUTPUT:		CASSETTE I/O

2-CONTROL PORTS FOR GAME PADDLES

CARTRIDGE EXPANSION SLOT

24-PIN USER I/O PORT

6-PIN SERIAL I/O CONNECTION

RF MODULATOR OUTPUT FOR TV DISPLAY

NTSC COMPOSITE COLOR OUTPUT FOR MONITOR

BIBLIOGRAPHY:

------------

1. "Beyond Games: Systems Software for Your 6502 Personal Computer" by Ken

Skier 1981.  This book was intended for the C= PET 2001 Computer.

2. "Machine Language for Beginners" by Richard Mansfield, 1983.  This book

was intended for the Atari, VIC, Apple, Commodore 64, and PET/CBM

computers.

3. "Assembly Language Programming with the Commodore 64" by Marvin L. De

Jong, 1984.

4. "Commodore 64 Troubleshooting & Repair Guide" by Robert C. Brenner, 1985.

5. "The Commodore 64 Programmer's Reference Guide" by CBM, 19xx.

6. "The Commodore 64 User's Guide" by CBM, 19xx.

7. "CHACKING MAG" (C) 1992 by Craig Taylor

8. "The PC Assembler Tutor" (C) 1989 by Chuck Nelson.